Chapter 11

# Response Surface Methods and Designs

Solutions

11.1S.	A chemical plant produces oxygen by liquefying air and separating it into its component gases by fractional distillation.  The purity of the oxygen is a function of the main condenser temperature and the pressure ratio between the upper and lower columns.   Current operating conditions are temperature  -220°C and pressure ratio 1.2.  Using the following data find the path of steepest ascent.

| Temperature | Pressure Ratio | Purity |
| --- | --- | --- |
| -225 | 1.1 | 82.8 |
| -225 | 1.3 | 83.5 |
| -215 | 1.1 | 84.7 |
| -215 | 1.3 | 85.0 |
| -220 | 1.2 | 84.1 |
| -220 | 1.2 | 84.5 |
| -220 | 1.2 | 83.9 |
| -220 | 1.2 | 84.3 |

Design Expert Output

Response:	Purity

ANOVA for Selected Factorial Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	3.14	2	1.57	26.17	0.0050	significant

A	2.89	1	2.89	48.17	0.0023

B	0.25	1	0.25	4.17	0.1108

Curvature	0.080	1	0.080	1.33	0.3125	not significant

Residual	0.24	4	0.060

Lack of Fit	0.040	1	0.040	0.60	0.4950	not significant

Pure Error	0.20	3	0.067

Cor Total	3.46	7

The Model F-value of 26.17 implies the model is significant.  There is only

a 0.50% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	0.24		R-Squared	0.9290

Mean	84.10		Adj R-Squared	0.8935

C.V.	0.29		Pred R-Squared	0.7123

PRESS	1.00		Adeq Precision	12.702

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	84.00	1	0.12	83.66	84.34

A-Temperature	0.85	1	0.12	0.51	1.19	1.00

B-Pressure Ratio	0.25	1	0.12	-0.090	0.59	1.00

Center Point	0.20	1	0.17	-0.28	0.68	1.00

Final Equation in Terms of Coded Factors:

Purity	 =

+84.00

+0.85	  * A

+0.25	  * B

Final Equation in Terms of Actual Factors:

Purity	 =

+118.40000

+0.17000	  * Temperature

+2.50000	  * Pressure Ratio

From the computer output use the model as the equation for steepest ascent.  Suppose we use a one degree change in temperature as the basic step size.  Thus, the path of steepest ascent passes through the point (x1=0, x2=0) and has a slope 0.25/0.85.  In the coded variables, one degree of temperature is equivalent to a step of . Thus, .  The path of steepest ascent is:

|  | Coded | Variables | Natural | Variables |
| --- | --- | --- | --- | --- |
|  | x1 | x2 |  |  |
| Origin | 0 | 0 | -220 | 1.2 |
|  | 0.2 | 0.059 | 1 | 0.0059 |
| Origin + | 0.2 | 0.059 | -219 | 1.2059 |
| Origin +5 | 1.0 | 0.295 | -215 | 1.2295 |
| Origin +7 | 1.40 | 0.413 | -213 | 1.2413 |

11.2.	Verify that the following design is a simplex.  Fit the first-order model and find the path of steepest ascent.

| Position | x1 | x2 | x3 | y |
| --- | --- | --- | --- | --- |
| 1 | 0 |  | -1 | 18.5 |
| 2 |  | 0 | 1 | 19.8 |
| 3 | 0 |  | -1 | 17.4 |
| 4 |  | 0 | 1 | 22.5 |

The graphical representation of the design identifies a tetrahedron; therefore, the design is a simplex.

Design Expert Output

Response:	y

ANOVA for Selected Factorial Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	14.49	3	4.83

A	3.64	1	3.64

B	0.61	1	0.61

C	10.24	1	10.24

Pure Error	0.000	0

Cor Total	14.49	3

Std. Dev.			R-Squared	1.0000

Mean	19.55		Adj R-Squared

C.V.			Pred R-Squared	N/A

PRESS	N/A		Adeq Precision	0.000

Case(s) with leverage of 1.0000:  Pred R-Squared and PRESS statistic not defined

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	19.55	1

A-x1	1.35	1				1.00

B-x2	0.55	1				1.00

C-x3	1.60	1				1.00

Final Equation in Terms of Coded Factors:

y	 =

+19.55

+1.35	  * A

+0.55	  * B

+1.60	  * C

Final Equation in Terms of Actual Factors:

y	 =

+19.55000

+0.95459	  * x1

+0.38891	  * x2

+1.60000	  * x3

The first order model is .

To find the path of steepest ascent, let the basic step size be .  Then using the results obtained in the previous problem, we obtain

or

which yields .  Then the coordinates of points on the path of steepest ascent are defined by

Therefore, in the coded variables we have:

|  | Coded | Variables |  |
| --- | --- | --- | --- |
|  | x1 | x2 | x3 |
| Origin | 0 | 0 | 0 |
|  | 0.84 | 0.34 | 1.00 |
| Origin + | 0.84 | 0.34 | 1.00 |
| Origin +2 | 1.68 | 0.68 | 2.00 |

11.3.	For the first-order model  find the path of steepest ascent.  The variables are coded as .

Let the basic step size be .    or  .  Then

Therefore, in the coded variables we have

|  | Coded | Variables |  |
| --- | --- | --- | --- |
|  | x1 | x2 | x3 |
| Origin | 0 | 0 | 0 |
|  | 0.75 | -0.40 | 1.00 |
| Origin + | 0.75 | -0.40 | 1.00 |
| Origin +2 | 1.50 | -0.80 | 2.00 |

11.4S. 	The region of experimentation for two factors are temperature  and catalyst feed rate .  A first order model in the usual  coded variables has been fit to a molecular weight response, yielding the following model.

(a)	Find the path of steepest ascent.

|  | Coded | Variables | Natural | Variables |
| --- | --- | --- | --- | --- |
|  | x1 | x2 | T | C |
| Origin | 0 | 0 | 200 | 20 |
|  | 1 | 0.32 | 100 | 3.2 |
| Origin + | 1 | 0.32 | 300 | 23.2 |
| Origin +5 | 5 | 1.60 | 700 | 36.0 |

(b)	It is desired to move to a region where molecular weights are above 2500.  Based on the information you have from the experimentation in this region, about how many steps along the path of steepest ascent might be required to move to the region of interest?

11.5.	The path of steepest ascent is usually computed assuming that the model is truly first-order.; that is, there is no interaction.  However, even if there is interaction, steepest ascent ignoring the interaction still usually produces good results.  To illustrate, suppose that we have fit the model

using coded variables (–1  x1  +1)

(a)	Draw the path of steepest ascent that you would obtain if the interaction were ignored.

(b)	Draw the path of steepest ascent that you would obtain with the interaction included in the model.  Compare this with the path found in part (a).

11.6.	The data shown in Table P11.2 were collected in an experiment to optimize crystal growth as a function of three variables x1, x2, and x3.  Large values of y (yield in grams) are desirable.  Fit a second order model and analyze the fitted surface.  Under what set of conditions is maximum growth achieved?

Table P11.2

| x1 | x2 | x3 | y |
| --- | --- | --- | --- |
| -1 | -1 | -1 | 66 |
| -1 | -1 | 1 | 70 |
| -1 | 1 | -1 | 78 |
| -1 | 1 | 1 | 60 |
| 1 | -1 | -1 | 80 |
| 1 | -1 | 1 | 70 |
| 1 | 1 | -1 | 100 |
| 1 | 1 | 1 | 75 |
| -1.682 | 0 | 0 | 100 |
| 1.682 | 0 | 0 | 80 |
| 0 | -1.682 | 0 | 68 |
| 0 | 1.682 | 0 | 63 |
| 0 | 0 | -1.682 | 65 |
| 0 | 0 | 1.682 | 82 |
| 0 | 0 | 0 | 113 |
| 0 | 0 | 0 | 100 |
| 0 | 0 | 0 | 118 |
| 0 | 0 | 0 | 88 |
| 0 | 0 | 0 | 100 |
| 0 | 0 | 0 | 85 |

Design Expert Output

Response:	Yield

ANOVA for Response Surface Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	3662.00	9	406.89	2.19	0.1194	not significant

A	22.08	1	22.08	0.12	0.7377

B	25.31	1	25.31	0.14	0.7200

C	30.50	1	30.50	0.16	0.6941

A2	204.55	1	204.55	1.10	0.3191

B2	2226.45	1	2226.45	11.96	0.0061

C2	1328.46	1	1328.46	7.14	0.0234

AB	66.12	1	66.12	0.36	0.5644

AC	55.13	1	55.13	0.30	0.5982

BC	171.13	1	171.13	0.92	0.3602

Residual	1860.95	10	186.09

Lack of Fit	1001.61	5	200.32	1.17	0.4353	not significant

Pure Error	859.33	5	171.87

Cor Total	5522.95	19

The "Model F-value" of 2.19 implies the model is not significant relative to the noise.  There is a

11.94 % chance that a "Model F-value" this large could occur due to noise.

Std. Dev.	13.64		R-Squared	0.6631

Mean	83.05		Adj R-Squared	0.3598

C.V.	16.43		Pred R-Squared	-0.6034

PRESS	8855.23		Adeq Precision	3.882

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	100.67	1	5.56	88.27	113.06

A-x1	1.27	1	3.69	-6.95	9.50	1.00

B-x2	1.36	1	3.69	-6.86	9.59	1.00

C-x3	-1.49	1	3.69	-9.72	6.73	1.00

A2	-3.77	1	3.59	-11.77	4.24	1.02

B2	-12.43	1	3.59	-20.44	-4.42	1.02

C2	-9.60	1	3.59	-17.61	-1.59	1.02

AB	2.87	1	4.82	-7.87	13.62	1.00

AC	-2.63	1	4.82	-13.37	8.12	1.00

BC	-4.63	1	4.82	-15.37	6.12	1.00

Final Equation in Terms of Coded Factors:

Yield	 =

+100.67

+1.27	  * A

+1.36	  * B

-1.49	  * C

-3.77	  * A2

-12.43	  * B2

-9.60	  * C2

+2.87	  * A * B

-2.63	  * A * C

-4.63	  * B * C

Final Equation in Terms of Actual Factors:

Yield	 =

+100.66609

+1.27146	  * x1

+1.36130	  * x2

-1.49445	  * x3

-3.76749	  * x12

-12.42955	  * x22

-9.60113	  * x32

+2.87500	  * x1 * x2

-2.62500	  * x1 * x3

-4.62500	  * x2 * x3

There are so many non-significant terms in this model that we should consider eliminating some of them.  A reasonable reduced model is shown below.

Design Expert Output

Response:	Yield

ANOVA for Response Surface Reduced Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	3143.00	4	785.75	4.95	0.0095	significant

B	25.31	1	25.31	0.16	0.6952

C	30.50	1	30.50	0.19	0.6673

B2	2115.31	1	2115.31	13.33	0.0024

C2	1239.17	1	1239.17	7.81	0.0136

Residual	2379.95	15	158.66

Lack of Fit	1520.62	10	152.06	0.88	0.5953	not significant

Pure Error	859.33	5	171.87

Cor Total	5522.95	19

The Model F-value of 4.95 implies the model is significant.  There is only

a 0.95% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	12.60		R-Squared	0.5691

Mean	83.05		Adj R-Squared	0.4542

C.V.	15.17		Pred R-Squared	0.1426

PRESS	4735.52		Adeq Precision	5.778

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	97.58	1	4.36	88.29	106.88

B-x2	1.36	1	3.41	-5.90	8.63	1.00

C-x3	-1.49	1	3.41	-8.76	5.77	1.00

B2	-12.06	1	3.30	-19.09	-5.02	1.01

C2	-9.23	1	3.30	-16.26	-2.19	1.01

Final Equation in Terms of Coded Factors:

Yield	 =

+97.58

+1.36	  * B

-1.49	  * C

-12.06	  * B2

-9.23	  * C2

Final Equation in Terms of Actual Factors:

Yield	 =

+97.58260

+1.36130	  * x2

-1.49445	  * x3

-12.05546	  * x22

-9.22703	  * x32

The contour plot identifies a maximum near the center of the design space.

11.7.	The data in Table P11.3 were collected by a chemical engineer.  The response y is filtration time, x1 is temperature, and x2 is pressure.  Fit a second-order model.

Table P11.3

| x1 | x2 | y |
| --- | --- | --- |
| -1 | -1 | 54 |
| -1 | 1 | 45 |
| 1 | -1 | 32 |
| 1 | 1 | 47 |
| -1.414 | 0 | 50 |
| 1.414 | 0 | 53 |
| 0 | -1.414 | 47 |
| 0 | 1.414 | 51 |
| 0 | 0 | 41 |
| 0 | 0 | 39 |
| 0 | 0 | 44 |
| 0 | 0 | 42 |
| 0 | 0 | 40 |

Design Expert Output

Response:	y

ANOVA for Response Surface Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	273.41	4	68.35	2.78	0.1018	not significant

A	31.04	1	31.04	1.26	0.2937

B	16.99	1	16.99	0.69	0.4299

A2	81.39	1	81.39	3.31	0.1063

AB	144.00	1	144.00	5.86	0.0418

Residual	196.59	8	24.57

Lack of Fit	181.79	4	45.45	12.28	0.0162	significant

Pure Error	14.80	4	3.70

Cor Total	470.00	12

The "Model F-value" of 2.78 implies the model is not significant relative to the noise.  There is a

10.18 % chance that a "Model F-value" this large could occur due to noise.

Std. Dev.	4.96	R-Squared	0.5817

Mean	45.00	Adj R-Squared	0.3726

C.V.	11.02	Pred R-Squared	-0.7171

PRESS	807.05	Adeq Precision	5.185

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	42.91	1	1.79	38.78	47.04

A-Temperature	-2.79	1	2.48	-8.50	2.93	1.00

B-Pressure	2.06	1	2.48	-3.65	7.78	1.00

A2	6.78	1	3.73	-1.81	15.38	1.00

AB	12.00	1	4.96	0.57	23.43	1.00

Final Equation in Terms of Coded Factors:

Time	 =

+42.91

+1.28	  * A

-1.79	  * B

+3.39	  * A2

+6.00	  * A * B

Time	 =

+42.91

-2.79	 * A

+2.06	 * B

+6.78	 * A2

+12.00	 * A * B

Final Equation in Terms of Actual Factors:

Time	 =

+42.91304

-1.96968	 * Temperature

+1.45711	 * Pressure

+3.39131	 * Temperature2

+6.00000	 * Temperature * Pressure

The lack of fit test in the above analysis is significant.  The residual plots below are not unusual; however, the three largest residuals are axial points.  Including cubic terms did not improve the lack of fit estimate and only made the PRESS larger.

(a)	What operating conditions would you recommend if the objective is to minimize the filtration time?

The contour plot below identifies the minimum filtration rate with the Temperature set at 1 and the Pressure at -1.  By setting the flag at this location in Design Expert, the prediction of 36.88 filtration rate can be quickly estimated.

(b)	What operating conditions would you recommend if the objective is to operate the process at a mean filtration time very close to 46?

The contour plot below identifies filtration time of 46 with the highlighted contours.  There are two regions where a filtration rate of 46 is achievable.  If higher temperature and pressure require higher operating costs, it would be preferential to choose the region with lower temperature and pressure.

11.8.	An experimenter has run a Box-Behnken design and has obtained the results as shown in Table P11.5, where the response variable is the viscosity of a polymer.

Table P11.5

| Level | Temp. | Agitation Rate | Pressure | x1 | x2 | x3 |
| --- | --- | --- | --- | --- | --- | --- |
| High | 200 | 10.0 | 25 | +1 | +1 | +1 |
| Middle | 175 | 7.5 | 20 | 0 | 0 | 0 |
| Low | 150 | 5.0 | 15 | -1 | -1 | -1 |

| Run | x1 | x2 | x3 | y1 |
| --- | --- | --- | --- | --- |
| 1 | -1 | -1 | 0 | 535 |
| 2 | 1 | -1 | 0 | 580 |
| 3 | -1 | 1 | 0 | 596 |
| 4 | 1 | 1 | 0 | 563 |
| 5 | -1 | 0 | -1 | 645 |
| 6 | 1 | 0 | -1 | 458 |
| 7 | -1 | 0 | 1 | 350 |
| 8 | 1 | 0 | 1 | 600 |
| 9 | 0 | -1 | -1 | 595 |
| 10 | 0 | 1 | -1 | 648 |
| 11 | 0 | -1 | 1 | 532 |
| 12 | 0 | 1 | 1 | 656 |
| 13 | 0 | 0 | 0 | 653 |
| 14 | 0 | 0 | 0 | 599 |
| 15 | 0 | 0 | 0 | 620 |

(a)	Fit the second-order model.

Design Expert Output

Response:	Viscosity

ANOVA for Response Surface Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	89652.58	9	9961.40	9.54	0.0115	significant

A	703.12	1	703.12	0.67	0.4491

B	6105.12	1	6105.12	5.85	0.0602

C	5408.00	1	5408.00	5.18	0.0719

A2	20769.23	1	20769.23	19.90	0.0066

B2	1404.00	1	1404.00	1.35	0.2985

C2	4719.00	1	4719.00	4.52	0.0868

AB	1521.00	1	1521.00	1.46	0.2814

AC	47742.25	1	47742.25	45.74	0.0011

BC	1260.25	1	1260.25	1.21	0.3219

Residual	5218.75	5	1043.75

Lack of Fit	3736.75	3	1245.58	1.68	0.3941	not significant

Pure Error	1482.00	2	741.00

Cor Total	94871.33	14

The Model F-value of 9.54 implies the model is significant.  There is only

a 1.15% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	32.31		R-Squared	0.9450

Mean	575.33		Adj R-Squared	0.8460

C.V.	5.62		Pred R-Squared	0.3347

PRESS	63122.50		Adeq Precision	10.425

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	624.00	1	18.65	576.05	671.95

A-Temp	9.37	1	11.42	-19.99	38.74	1.00

B-Agitation Rate	27.62	1	11.42	-1.74	56.99	1.00

C-Pressure	-26.00	1	11.42	-55.36	3.36	1.00

A2	-75.00	1	16.81	-118.22	-31.78	1.01

B2	19.50	1	16.81	-23.72	62.72	1.01

C2	-35.75	1	16.81	-78.97	7.47	1.01

AB	-19.50	1	16.15	-61.02	22.02	1.00

AC	109.25	1	16.15	67.73	150.77	1.00

BC	17.75	1	16.15	-23.77	59.27	1.00

Final Equation in Terms of Coded Factors:

Viscosity	 =

+624.00

+9.37	  * A

+27.62	  * B

-26.00	  * C

-75.00	  * A2

+19.50	  * B2

-35.75	  * C2

-19.50	  * A * B

+109.25	  * A * C

+17.75	  * B * C

Final Equation in Terms of Actual Factors:

Viscosity	 =

-629.50000

+27.23500	  * Temp

-9.55000	  * Agitation Rate

-111.60000	  * Pressure

-0.12000	  * Temp2

+3.12000	  * Agitation Rate2

-1.43000	  * Pressure2

-0.31200	  * Temp * Agitation Rate

+0.87400	  * Temp * Pressure

+1.42000	  * Agitation Rate * Pressure

(b)	Perform the canonical analysis.  What type of surface has been found?

#### Solution

### Variable	Critical Value

X1	2.1849596

X2	-0.871371

X3	2.7586015

Predicted Value at Solution 	586.34437

##### Eigenvalues and Eigenvectors

Variable	20.9229	2.5208	-114.694

X1	-0.02739	0.58118	0.81331

X2	0.99129	-0.08907	0.09703

X3	0.12883	0.80888	-0.57368

The system is a saddle point.

(c)	What operating conditions on x1, x2, and x3 lead to the stationary point?

The stationary point is (x1, x2, x3) = (2.18496, -0.87167, 2.75860).  This is outside the design region.  It would be necessary to either examine contour plots or use numerical optimization methods to find desired operating conditions.

(d)	What operating conditions would you recommend if it is important to obtain a viscosity that is as close to 600 as possible?

A: Temp

Any point on either of the contours showing a viscosity of 600 is satisfactory.

11.9S.	Consider the three-variable central composite design shown in Table P11.6.  Analyze the data and draw conclusions, assuming that we wish to maximize conversion (y1) with activity (y2) between 55 and 60.

Table P11.6

| Run | Time (min) | Temperature (C) | Catalyst (%) | Conversion (%) y1 | Activity y2 |
| --- | --- | --- | --- | --- | --- |
| 1 | -1.000 | -1.000 | -1.000 | 74.00 | 53.20 |
| 2 | 1.000 | -1.000 | -1.000 | 51.00 | 62.90 |
| 3 | -1.000 | 1.000 | -1.000 | 88.00 | 53.40 |
| 4 | 1.000 | 1.000 | -1.000 | 70.00 | 62.60 |
| 5 | -1.000 | -1.000 | 1.000 | 71.00 | 57.30 |
| 6 | 1.000 | -1.000 | 1.000 | 90.00 | 67.90 |
| 7 | -1.000 | 1.000 | 1.000 | 66.00 | 59.80 |
| 8 | 1.000 | 1.000 | 1.000 | 97.00 | 67.80 |
| 9 | 0.000 | 0.000 | 0.000 | 81.00 | 59.20 |
| 10 | 0.000 | 0.000 | 0.000 | 75.00 | 60.40 |
| 11 | 0.000 | 0.000 | 0.000 | 76.00 | 59.10 |
| 12 | 0.000 | 0.000 | 0.000 | 83.00 | 60.60 |
| 13 | -1.682 | 0.000 | 0.000 | 76.00 | 59.10 |
| 14 | 1.682 | 0.000 | 0.000 | 79.00 | 65.90 |
| 15 | 0.000 | -1.682 | 0.000 | 85.00 | 60.00 |
| 16 | 0.000 | 1.682 | 0.000 | 97.00 | 60.70 |
| 17 | 0.000 | 0.000 | -1.682 | 55.00 | 57.40 |
| 18 | 0.000 | 0.000 | 1.682 | 81.00 | 63.20 |
| 19 | 0.000 | 0.000 | 0.000 | 80.00 | 60.80 |
| 20 | 0.000 | 0.000 | 0.000 | 91.00 | 58.90 |

Quadratic models are developed for the Conversion and Activity response variables as follows:

Design Expert Output

Response:	Conversion

ANOVA for Response Surface Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	2555.73	9	283.97	12.76	0.0002	significant	A	14.44	1	14.44	0.65	0.4391

B	222.96	1	222.96	10.02	0.0101

C	525.64	1	525.64	23.63	0.0007

A2	48.47	1	48.47	2.18	0.1707

B2	124.48	1	124.48	5.60	0.0396

C2	388.59	1	388.59	17.47	0.0019

AB	36.13	1	36.13	1.62	0.2314

AC	1035.13	1	1035.13	46.53	< 0.0001

BC	120.12	1	120.12	5.40	0.0425

Residual	222.47	10	22.25

Lack of Fit	56.47	5	11.29	0.34	0.8692	not significant

Pure Error	166.00	5	33.20

Cor Total	287.28	19

The Model F-value of 12.76 implies the model is significant.  There is only

a 0.02% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	4.72		R-Squared	0.9199

Mean	78.30		Adj R-Squared	0.8479

C.V.	6.02		Pred R-Squared	0.7566

PRESS	676.22		Adeq Precision	14.239

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	81.09	1	1.92	76.81	85.38

A-Time	1.03	1	1.28	-1.82	3.87	1.00

B-Temperature	4.04	1	1.28	1.20	6.88	1.00

C-Catalyst	6.20	1	1.28	3.36	9.05	1.00

A2	-1.83	1	1.24	-4.60	0.93	1.02

B2	2.94	1	1.24	0.17	5.71	1.02

C2	-5.19	1	1.24	-7.96	-2.42	1.02

AB	2.13	1	1.67	-1.59	5.84	1.00

AC	11.38	1	1.67	7.66	15.09	1.00

BC	-3.87	1	1.67	-7.59	-0.16	1.00

Final Equation in Terms of Coded Factors:

Conversion	 =

+81.09

+1.03	 * A

+4.04	 * B

+6.20	 * C

-1.83	 * A2

+2.94	 * B2

-5.19	 * C2

+2.13	 * A * B

+11.38	 * A * C

-3.87	 * B * C

Final Equation in Terms of Actual Factors:

Conversion	 =

+81.09128

+1.02845	 * Time

+4.04057	 * Temperature

+6.20396	 * Catalyst

-1.83398	 * Time2

+2.93899	 * Temperature2

-5.19274	 * Catalyst2

+2.12500	 * Time * Temperature

+11.37500	 * Time * Catalyst

-3.87500	 * Temperature * Catalyst

Design Expert Output

Response:	Activity

ANOVA for Response Surface Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	256.20	9	28.47	9.16	0.0009	significant	A	175.35	1	175.35	56.42	< 0.0001

B	0.89	1	0.89	0.28	0.6052

C	67.91	1	67.91	21.85	0.0009

A2	10.05	1	10.05	3.23	0.1024

B2	0.081	1	0.081	0.026	0.8753

C2	0.047	1	0.047	0.015	0.9046

AB	1.20	1	1.20	0.39	0.5480

AC	0.011	1	0.011	3.620E-003	0.9532

BC	0.78	1	0.78	0.25	0.6270

Residual	31.08	10	3.11

Lack of Fit	27.43	5	5.49	7.51	0.0226	significant

Pure Error	3.65	5	0.73

Cor Total	287.28	19

The Model F-value of 9.16 implies the model is significant.  There is only

a 0.09% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	1.76		R-Squared	0.8918

Mean	60.51		Adj R-Squared	0.7945

C.V.	2.91		Pred R-Squared	0.2536

PRESS	214.43		Adeq Precision	10.911

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	59.85	1	0.72	58.25	61.45

A-Time	3.58	1	0.48	2.52	4.65	1.00

B-Temperature	0.25	1	0.48	-0.81	1.32	1.00

C-Catalyst	2.23	1	0.48	1.17	3.29	1.00

A2	0.83	1	0.46	-0.20	1.87	1.02

B2	0.075	1	0.46	-0.96	1.11	1.02

C2	0.057	1	0.46	-0.98	1.09	1.02

AB	-0.39	1	0.62	-1.78	1.00	1.00

AC	-0.038	1	0.62	-1.43	1.35	1.00

BC	0.31	1	0.62	-1.08	1.70	1.00

Final Equation in Terms of Coded Factors:

Conversion	 =

+59.85

+3.58	  * A

+0.25	  * B

+2.23	  * C

+0.83	  * A2

+0.075	  * B2

+0.057	  * C2

-0.39	  * A * B

-0.038	  * A * C

+0.31	  * B * C

Final Equation in Terms of Actual Factors:

Conversion	 =

+59.84984

+3.58327	  * Time

+0.25462	  * Temperature

+2.22997	  * Catalyst

+0.83491	  * Time2

+0.074772	  * Temperature2

+0.057094	  * Catalyst2

-0.38750	  * Time * Temperature

-0.037500	  * Time * Catalyst

+0.31250	  * Temperature * Catalyst

Because many of the terms are insignificant, the reduced quadratic model is fit as follows:

Design Expert Output

Response:	Activity

ANOVA for Response Surface Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	253.20	3	84.40	39.63	< 0.0001	significant

A	175.35	1	175.35	82.34	< 0.0001

C	67.91	1	67.91	31.89	< 0.0001

A2	9.94	1	9.94	4.67	0.0463

Residual	34.07	16	2.13

Lack of Fit	30.42	11	2.77	3.78	0.0766	not significant

Pure Error	3.65	5	0.73

Cor Total	287.28	19

The Model F-value of 39.63 implies the model is significant.  There is only

a 0.01% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	1.46		R-Squared	0.8814

Mean	60.51		Adj R-Squared	0.8591

C.V.	2.41		Pred R-Squared	0.6302

PRESS	106.24		Adeq Precision	20.447

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	59.95	1	0.42	59.06	60.83

A-Time	3.58	1	0.39	2.75	4.42	1.00

C-Catalyst	2.23	1	0.39	1.39	3.07	1.00

A2	0.82	1	0.38	0.015	1.63	1.00

Final Equation in Terms of Coded Factors:

Activity	 =

+59.95

+3.58	 * A

+2.23	 * C

+0.82	 * A2

Final Equation in Terms of Actual Factors:

Activity	 =

+59.94802

+3.58327	 * Time

+2.22997	 * Catalyst

+0.82300	 * Time2

The contour plots visually describe the models while the overlay plots identifies the acceptable region for the process.

11.10.	A manufacturer of cutting tools has developed two empirical equations for tool life in hours (y1) and for tool cost in dollars (y2).  Both models are linear functions of steel hardness (x1) and manufacturing time (x2).  The two equations are

and both equations are valid over the range -1.5xi1.5.  Unit tool cost must be below $27.50 and life must exceed 12 hours for the product to be competitive.  Is there a feasible set of operating conditions for this process?  Where would you recommend that the process be run?

The contour plots below graphically describe the two models.  The overlay plot identifies the feasible operating region for the process.

11.11.	A central composite design is run in a chemical vapor deposition process, resulting in the experimental data shown in Table P11.7.  Four experimental units were processed simultaneously on each run of the design, and the responses are the mean and variance of thickness, computed across the four units.

Table P11.7

| x1 | x2 |  | s2 |
| --- | --- | --- | --- |
| -1 | -1 | 360.6 | 6.689 |
| 1 | -1 | 445.2 | 14.230 |
| -1 | 1 | 412.1 | 7.088 |
| 1 | 1 | 601.7 | 8.586 |
| 1.414 | 0 | 518.0 | 13.130 |
| -1.414 | 0 | 411.4 | 6.644 |
| 0 | 1.414 | 497.6 | 7.649 |
| 0 | -1.414 | 397.6 | 11.740 |
| 0 | 0 | 530.6 | 7.836 |
| 0 | 0 | 495.4 | 9.306 |
| 0 | 0 | 510.2 | 7.956 |
| 0 | 0 | 487.3 | 9.127 |

(a)	Fit a model to the mean response.  Analyze the residuals.

Design Expert Output

Response:	Mean Thick

ANOVA for Response Surface Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	47644.26	5	9528.85	16.12	0.0020	significant

A	22573.36	1	22573.36	38.19	0.0008

B	15261.91	1	15261.91	25.82	0.0023

A2	2795.58	1	2795.58	4.73	0.0726

B2	5550.74	1	5550.74	9.39	0.0221

AB	2756.25	1	2756.25	4.66	0.0741

Residual	3546.83	6	591.14

Lack of Fit	2462.04	3	820.68	2.27	0.2592	not significant

Pure Error	1084.79	3	361.60

Cor Total	51191.09	11

The Model F-value of 16.12 implies the model is significant.  There is only

a 0.20% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	24.31		R-Squared	0.9307

Mean	472.31		Adj R-Squared	0.8730

C.V.	5.15		Pred R-Squared	0.6203

PRESS	19436.37		Adeq Precision	11.261

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	505.88	1	12.16	476.13	535.62

A-x1	53.12	1	8.60	32.09	74.15	1.00

B-x2	43.68	1	8.60	22.64	64.71	1.00

A2	-20.90	1	9.61	-44.42	2.62	1.04

B2	-29.45	1	9.61	-52.97	-5.93	1.04

AB	26.25	1	12.16	-3.50	56.00	1.00

Final Equation in Terms of Coded Factors:

Mean Thick	 =

+505.88

+53.12	  * A

+43.68	  * B

-20.90	  * A2

-29.45	  * B2

+26.25	  * A * B

Final Equation in Terms of Actual Factors:

Mean Thick	 =

+505.87500

+53.11940	  * x1

+43.67767	  * x2

-20.90000	  * x12

-29.45000	  * x22

+26.25000	  * x1 * x2

A modest deviation from normality can be observed in the Normal Plot of Residuals; however, not enough to be concerned.

(b)	Fit a model to the variance response.  Analyze the residuals.

Design Expert Output

Response:	Var Thick

ANOVA for Response Surface 2FI Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	65.80	3	21.93	35.86	< 0.0001	significant

A	41.46	1	41.46	67.79	< 0.0001

B	15.21	1	15.21	24.87	0.0011

AB	9.13	1	9.13	14.93	0.0048

Residual	4.89	8	0.61

Lack of Fit	3.13	5	0.63	1.06	0.5137	not significant

Pure Error	1.77	3	0.59

Cor Total	70.69	11

The Model F-value of 35.86 implies the model is significant.  There is only

a 0.01% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	0.78		R-Squared	0.9308

Mean	9.17		Adj R-Squared	0.9048

C.V.	8.53		Pred R-Squared	0.8920

PRESS	7.64		Adeq Precision	18.572

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	9.17	1	0.23	8.64	9.69

A-x1	2.28	1	0.28	1.64	2.91	1.00

B-x2	-1.38	1	0.28	-2.02	-0.74	1.00

AB	-1.51	1	0.39	-2.41	-0.61	1.00

Final Equation in Terms of Coded Factors:

Var Thick	 =

+9.17

+2.28	  * A

-1.38	  * B

-1.51	  * A * B

Final Equation in Terms of Actual Factors:

Var Thick	 =

+9.16508

+2.27645	  * x1

-1.37882	  * x2

-1.51075	  * x1 * x2

The residual plots are not acceptable.  A transformation should be considered.  If not successful at correcting the residual plots, further investigation into the two apparently unusual points should be made.

(c)	Fit a model to the ln(s2).  Is this model superior to the one you found in part (b)?

Design Expert Output

Response:	Var Thick	Transform:	Natural log	Constant:	0

ANOVA for Response Surface 2FI Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	0.67	3	0.22	36.94	< 0.0001	significant

A	0.46	1	0.46	74.99	< 0.0001

B	0.14	1	0.14	22.80	0.0014

AB	0.079	1	0.079	13.04	0.0069

Residual	0.049	8	6.081E-003

Lack of Fit	0.024	5	4.887E-003	0.61	0.7093	not significant

Pure Error	0.024	3	8.071E-003

Cor Total	0.72	11

The Model F-value of 36.94 implies the model is significant.  There is only

a 0.01% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	0.078		R-Squared	0.9327

Mean	2.18		Adj R-Squared	0.9074

C.V.	3.57		Pred R-Squared	0.8797

PRESS	0.087		Adeq Precision	18.854

Coefficient		Standard	95% CI	95% CI

Factor	Estimate	DF	Error	Low	High	VIF

Intercept	2.18	1	0.023	2.13	2.24

A-x1	0.24	1	0.028	0.18	0.30	1.00

B-x2	-0.13	1	0.028	-0.20	-0.068	1.00

AB	-0.14	1	0.039	-0.23	-0.051	1.00

Final Equation in Terms of Coded Factors:

Ln(Var Thick)	 =

+2.18

+0.24	  * A

-0.13	  * B

-0.14	  * A * B

Final Equation in Terms of Actual Factors:

Ln(Var Thick)	 =

+2.18376

+0.23874	  * x1

-0.13165	  * x2

-0.14079	  * x1 * x2

The residual plots are much improved following the natural log transformation; however, the two runs still appear to be somewhat unusual and should be investigated further.  They will be retained in the analysis.

(d)	Suppose you want the mean thickness to be in the interval 450 ± 25.  Find a set of operating conditions that achieve the objective and simultaneously minimize the variance.

The contour plots describe the two models while the overlay plot identifies the acceptable region for the process.

(e)	Discuss the variance minimization aspects of part (d).  Have you minimized total process variance?

The within run variance has been minimized; however, the run-to-run variation has not been minimized in the analysis.  This may not be the most robust operating conditions for the process.

11.12S.	Verify that an orthogonal first-order design is also first-order rotatable.

To show that a first order orthogonal design is also first order rotatable, consider

since all covariances between  and are zero, due to design orthogonality. Furthermore, we have:

, so

which is a function of distance from the design center (i.e. x=0), and not direction.  Thus the design must be rotatable.  Note that n is, in general, the number of points in the exterior portion of the design.  If there are nc center points, then .

11.13.	Show that augmenting a 2k design with nc center points does not affect the estimates of the i (i=1, 2, . . . , k), but that the estimate of the intercept 0 is the average of all 2k + nc observations.

In general, the X matrix for the 2k design with nc center points and the y vector would be:

|  | The upper half of the matrix is the usual 1 notation of the 2k design The lower half of the matrix represents the center points (nc rows) |
| --- | --- |

|  | 2k+nc observations. |  |  |  | Grand total of all 2k+nc observations usual contrasts from 2k |
| --- | --- | --- | --- | --- | --- |

Therefore, , which is the average of all observations, while , which does not depend on the number of center points, since in computing the contrasts gi, all observations at the center are multiplied by zero.

11.14S.	Verify that the central composite design shown in Table P11.8 blocks orthogonally.

Table P11.8

|  | Block 1 |  |  | Block 2 |  |  | Block 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x1 | x2 | x3 | x1 | x2 | x3 | x1 | x2 | x3 |
| 0 | 0 | 0 | 0 | 0 | 0 | -1.633 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1.633 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | -1 | 0 | -1.633 | 0 |
| 1 | -1 | -1 | 1 | -1 | 1 | 0 | 1.633 | 0 |
| -1 | -1 | 1 | -1 | 1 | 1 | 0 | 0 | -1.633 |
| -1 | 1 | -1 | -1 | -1 | -1 | 0 | 0 | 1.633 |
|  |  |  |  |  |  | 0 | 0 | 0 |
|  |  |  |  |  |  | 0 | 0 | 0 |

Note that each block is an orthogonal first order design, since the cross products of elements in different columns add to zero for each block.  To verify the second condition, choose a column, say column x2.  Now

, and n = 20

For blocks 1 and 2,

and condition 2 is satisfied by blocks 1 and 2.  For block 3, we have

And condition 2 is satisfied by block 3.  Similar results hold for the other columns.

11.15.	Blocking in the central composite design.  Consider a central composite design for k = 4 variables in two blocks.  Can a rotatable design always be found that blocks orthogonally?

To run a central composite design in two blocks, assign the nf factorial points and the nc1 center points to block 1 and the 2k axial points plus nc2 center points to block 2.  Both blocks will be orthogonal first order designs, so the first condition for orthogonal blocking is satisfied.

The second condition implies that

However, in block 1 and in block 2, so

Which gives:

Since  if the design is to be rotatable, then the design must satisfy

It is not possible to find rotatable central composite designs which block orthogonally for all k.  For example, if k=3, the above condition cannot be satisfied.  For k=2, there must be an equal number of center points in each block, i.e. nc1 = nc2.  For k=4, we must have nc1 = 4 and nc2 = 2.

11.16.	How could a hexagon design be run in two orthogonal blocks?

The hexagonal design can be blocked as shown below.  There are nc1 = nc2 = nc center points with nc even.

Put the points 1,3,and 5 in block 1 and 2,4,and 6 in block 2.  Note that each block is a simplex.

11.17.	Yield during the first four cycles of a chemical process is shown in the following table.  The variables are percent concentration (x1) at levels 30, 31, and 32 and temperature (x2) at 140, 142, and 144F.  Analyze by EVOP methods.

|  |  |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
| Cycle | (1) | (2) | (3) | (4) | (5) |
| 1 | 60.7 | 59.8 | 60.2 | 64.2 | 57.5 |
| 2 | 59.1 | 62.8 | 62.5 | 64.6 | 58.3 |
| 3 | 56.6 | 59.1 | 59.0 | 62.3 | 61.1 |
| 4 | 60.5 | 59.8 | 64.5 | 61.0 | 60.1 |

Cycle:   n=1  Phase 1

| Calculation of Averages |  |  |  |  |  |  |  | Calculation of Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Conditions |  |  | (1) | (2) | (3) | (4) | (5) |  |
| (i) | Previous Cycle Sum |  |  |  |  |  |  | Previous Sum S= |
| (ii) | Previous Cycle Average |  |  |  |  |  |  | Previous Average = |
| (iii) | New Observation |  | 60.7 | 59.8 | 60.2 | 64.2 | 57.5 | New S=Range x fk,n |
| (iv) | Differences |  |  |  |  |  |  | Range= |
| (v) | New Sums |  | 60.7 | 59.8 | 60.2 | 64.2 | 57.5 | New Sum S= |
| (vi) | New Averages |  | 60.7 | 59.8 | 60.2 | 64.2 | 57.5 | New average S = New Sum S/(n-1)= |

| Calculation of Effects |  | Calculation of Error Limits |  |
| --- | --- | --- | --- |

Cycle:   n=2  Phase 1

| Calculation of Averages |  |  |  |  |  |  |  | Calculation of Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Conditions |  |  | (1) | (2) | (3) | (4) | (5) |  |
| (i) | Previous Cycle Sum |  | 60.7 | 59.8 | 60.2 | 64.2 | 57.5 | Previous Sum S= |
| (ii) | Previous Cycle Average |  | 60.7 | 59.8 | 60.2 | 64.2 | 57.5 | Previous Average = |
| (iii) | New Observation |  | 59.1 | 62.8 | 62.5 | 64.6 | 58.3 | New S=Range x fk,n=1.38 |
| (iv) | Differences |  | 1.6 | -3.0 | -2.3 | -0.4 | -0.8 | Range=4.6 |
| (v) | New Sums |  | 119.8 | 122.6 | 122.7 | 128.8 | 115.8 | New Sum S=1.38 |
| (vi) | New Averages |  | 59.90 | 61.30 | 61.35 | 64.40 | 57.90 | New average S = New Sum S/(n-1)=1.38 |

| Calculation of Effects |  | Calculation of Error Limits |
| --- | --- | --- |

Cycle:   n=3  Phase 1

| Calculation of Averages |  |  |  |  |  |  |  |  |  |  |  |  |  | Calculation of Standard Deviation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Conditions |  |  |  | (1) |  | (2) |  | (3) |  | (4) |  | (5) |  |  |  |
| (i) | Previous Cycle Sum |  | 119.8 |  | 122.6 |  | 122.7 |  | 128.8 |  | 115.8 |  | Previous Sum S=1.38 |  |
| (ii) | Previous Cycle Average |  | 59.90 |  | 61.30 |  | 61.35 |  | 64.40 |  | 57.90 |  | Previous Average =1.38 |  |
| (iii) | New Observation |  | 56.6 |  | 59.1 |  | 59.0 |  | 62.3 |  | 61.1 |  | New S=Range x fk,n=2.28 |  |
| (iv) | Differences |  | 3.30 |  | 2.20 |  | 2.35 |  | 2.10 |  | -3.20 |  | Range=6.5 |  |
| (v) | New Sums |  | 176.4 |  | 181.7 |  | 181.7 |  | 191.1 |  | 176.9 |  | New Sum S=3.66 |  |
| (vi) | New Averages |  | 58.80 |  | 60.57 |  | 60.57 |  | 63.70 |  | 58.97 |  | New average S = New Sum S/(n-1)=1.83 |  |

| Calculation of Effects |  | Calculation of Error Limits |  |
| --- | --- | --- | --- |
|  | 2.37 | For New Average: | 2.11 |
|  | -2.37 | For New Effects: | 2.11 |
|  | -0.77 | For CIM: | 1.88 |
|  | 1.72 |  |  |

Cycle:   n=4  Phase 1

| Calculation of Averages |  |  |  |  |  |  |  | Calculation of Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Conditions |  |  | (1) | (2) | (3) | (4) | (5) |  |
| (i) | Previous Cycle Sum |  | 176.4 | 181.7 | 181.7 | 191.1 | 176.9 | Previous Sum S=3.66 |
| (ii) | Previous Cycle Average |  | 58.80 | 60.57 | 60.57 | 63.70 | 58.97 | Previous Average =1.83 |
| (iii) | New Observation |  | 60.5 | 59.8 | 64.5 | 61.0 | 60.1 | New S=Range x fk,n=2.45 |
| (iv) | Differences |  | -1.70 | 0.77 | -3.93 | 2.70 | -1.13 | Range=6.63 |
| (v) | New Sums |  | 236.9 | 241.5 | 246.2 | 252.1 | 237.0 | New Sum S=6.11 |
| (vi) | New Averages |  | 59.23 | 60.38 | 61.55 | 63.03 | 59.25 | New average S = New Sum S/(n-1)=2.04 |

| Calculation of Effects |  | Calculation of Error Limits |  |
| --- | --- | --- | --- |
|  | 2.48 | For New Average: | 2.04 |
|  | -1.31 | For New Effects: | 2.04 |
|  | -0.18 | For CIM: | 1.82 |
|  | 1.46 |  |  |

From studying cycles 3 and 4, it is apparent that A (and possibly B) has a significant effect.  A new phase should be started following cycle 3 or 4.

11.18.	Suppose that we approximate a response surface with a model of order d1, such as y=X11+, when the true surface is described by a model of order d2>d1; that is E(y) = X11+ X22.

(a)	Show that the regression coefficients are biased, that is, that E()=1+A2, where

A = (X’1X1)-1X’1X2. A is usually called the alias matrix.

(b)	If d1=1 and d2=2, and a full 2k is used to fit the model, use the result in part (a) to determine the alias structure.

In this situation, we have assumed the true surface to be first order, when it is really second order.  If a full factorial is used for k=2, then

The pure quadratic terms bias the intercept.

(c)	If d1=1, d2=2 and k=3, find the alias structure assuming that a 23-1 design is used to fit the model.

(d)	If d1=1, d2=2, k=3, and the simplex design in Problem 11.3 is used to fit the model, determine the alias structure and compare the results with part (c).

Notice that the alias structure is different from that found in the previous part for the 23-1 design.  In general, the A matrix will depend on which simplex design is used.

11.19S.	Suppose that you need to design an experiment to fit a quadratic model over the region , i = 1, 2   subject to the constraint .  If the constraint is violated, the process will not work properly.  You can afford to make no more than n = 12 runs.  Set up the following designs:

(a)	An “inscribed” CCD with center points at

| x1 | x2 |
| --- | --- |
| -0.5 | -0.5 |
| 0.5 | -0.5 |
| -0.5 | 0.5 |
| 0.5 | 0.5 |
| -0.707 | 0 |
| 0.707 | 0 |
| 0 | -0.707 |
| 0 | 0.707 |
| 0 | 0 |
| 0 | 0 |
| 0 | 0 |
| 0 | 0 |

(a)*	An “inscribed” CCD with center points at  so that a larger design could be fit within the constrained region

| x1 | x2 |
| --- | --- |
| -1 | -1 |
| 0.5 | -1 |
| -1 | 0.5 |
| 0.5 | 0.5 |
| -1.664 | -0.25 |
| 1.164 | -0.25 |
| -0.25 | -1.664 |
| -0.25 | 1.164 |
| -0.25 | -0.25 |
| -0.25 | -0.25 |
| -0.25 | -0.25 |
| -0.25 | -0.25 |

(b)	An “inscribed” 32 factorial with center points at

| x1 | x2 |
| --- | --- |
| -1 | -1 |
| -0.25 | -1 |
| 0.5 | -1 |
| -1 | -0.25 |
| -0.25 | -0.25 |
| 0.5 | -0.25 |
| -1 | 0.5 |
| -0.25 | 0.5 |
| 0.5 | 0.5 |
| -0.25 | -0.25 |
| -0.25 | -0.25 |
| -0.25 | -0.25 |

(c)	A D-optimal design.

| x1 | x2 |
| --- | --- |
| -1 | -1 |
| 1 | -1 |
| -1 | 1 |
| 1 | 0 |
| 0 | 1 |
| 0 | 0 |
| -1 | 0 |
| 0 | -1 |
| 0.5 | 0.5 |
| -1 | -1 |
| 1 | -1 |
| -1 | 1 |

(d)	A modified D-optimal design that is identical to the one in part (c), but with all replicate runs at the design center.

| x1 | x2 |
| --- | --- |
| 1 | 0 |
| 0 | 0 |
| 0 | 1 |
| -1 | -1 |
| 1 | -1 |
| -1 | 1 |
| -1 | 0 |
| 0 | -1 |
| 0.5 | 0.5 |
| 0 | 0 |
| 0 | 0 |
| 0 | 0 |

(e)	Evaluate the  criteria for each design.

|  | (a) | (a)* | (b) | (c) | (d) |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 | 0.00005248 | 0.007217 | 0.0001016 | 0.0002294 |

(f)	Evaluate the D-efficiency for each design relative to the D-optimal design in part (c).

|  | (a) | (a)* | (b) | (c) | (d) |
| --- | --- | --- | --- | --- | --- |
| D-efficiency | 24.25% | 111.64% | 49.14% | 100.00% | 87.31% |

(g)	Which design would you prefer?  Why?

The offset CCD, (a)*, is the preferred design based on the D-efficiency.  Not only is it better than the D-optimal design, (c), but it maintains the desirable design features of the CCD.

11.20S.	Consider a 23 design for fitting a first-order model.

(a)	Evaluate the D-criterion  for this design.

(b)	Evaluate the A-criterion   for this design.

(c)	Find the maximum scaled prediction variance for this design.  Is this design G-optimal?

.

Yes, this is a G-optimal design.

11.21.	Repeat Problem 11.20 using a first order model with the two-factor interaction.

.

Yes, this is a G-optimal design.

11.22S.	Suppose that you want to fit a second-order model in k = 5 factors.  You cannot afford more than 25 runs.  Construct both a D-optimal and an I-optimal design for this situation.  Compare the prediction variance properties of the designs.  Which design would you prefer?

The following JMP outputs identify both D-optimal and I-optimal designs.  The prediction variance profile shown in each JMP output is appreciably less for the I-optimal design as well as flatter in the middle of the design.

JMP Output

D-optimal Design

Factors

Add N Factors

1

X1   Continuous

X2   Continuous

X3   Continuous

X4   Continuous

X5   Continuous

Design

Run	X1	X2	X3	X4	X5

1	-1	-1	1	1	0

2	-1	1	1	-1	-1

3	-1	1	-1	-1	1

4	1	-1	-1	-1	1

5	-1	-1	-1	1	1

6	0	1	1	-1	0

7	0	-1	0	1	-1

8	-1	-1	-1	-1	-1

9	1	-1	-1	1	-1

10	1	-1	1	1	1

11	-1	-1	1	-1	1

12	1	-1	1	-1	-1

13	-1	1	-1	1	-1

14	1	1	1	1	-1

15	0	-1	-1	0	0

16	-1	0	0	-1	0

17	1	1	1	-1	1

18	1	1	0	0	-1

19	-1	-1	1	0	-1

20	-1	1	1	1	1

21	1	1	-1	1	1

22	1	1	-1	-1	-1

23	0	0	1	0	1

24	0	0	1	1	-1

25	1	0	-1	0	0

Prediction Variance Profile

Prediction Variance Surface

JMP Output

I-Optimal Design

Factors

Add N Factors

1

X1   Continuous

X2   Continuous

X3   Continuous

X4   Continuous

X5   Continuous

Design

Run	X1	X2	X3	X4	X5

1	-1	0	-1	1	0

2	1	1	1	-1	-1

3	1	1	-1	1	-1

4	0	-1	0	-1	0

5	-1	1	0	0	0

6	0	0	0	1	1

7	-1	0	1	-1	1

8	1	-1	-1	1	1

9	1	0	0	0	-1

10	1	0	-1	-1	0

11	-1	-1	-1	-1	1

12	-1	-1	1	-1	-1

13	1	-1	1	0	1

14	1	1	0	-1	1

15	0	1	1	1	-1

16	-1	0	0	0	-1

17	1	1	1	1	0

18	0	1	-1	0	1

19	1	-1	1	1	-1

20	0	0	0	0	0

21	-1	1	1	1	1

22	-1	1	-1	-1	-1

23	-1	-1	1	1	0

24	0	-1	-1	0	-1

25	0	0	1	0	0

Prediction Variance Profile

Prediction Variance Surface

11.23.	Suppose that you want to fit a second-order response surface model in a situation where there are k = 4 factors; however, one of the factors is categorical with two levels.  What model should you consider for this experiment?  Suggest an appropriate design for this situation.

The following model is considered for this experiment:

The following JMP output identifies a computer generated design utilizing the I-criterion.  Because this is a response surface model which is generally used for prediction, the I-criterion is an appropriate choice for a computer generated design.

JMP Output

Custom Design

Factors

Add N Factors

1

X1   Continuous

X2   Continuous

X3   Continuous

X4   Categorical

Model

Intercept

Intercept

X1

X2

X3

X4

X1*X2

X1*X3

X1*X4

X2*X3

X2*X4

X3*X4

X1*X1

X2*X2

X3*X3

Design

Run	X1	X2	X3	X4

1	0	0	0	L2

2	-1	-1	-1	L2

3	1	-1	1	L1

4	1	1	1	L2

5	0	-1	-1	L1

6	0	1	1	L1

7	-1	0	1	L1

8	0	1	-1	L2

9	-1	-1	0	L1

10	0	-1	1	L2

11	-1	1	-1	L1

12	1	-1	-1	L2

13	1	0	-1	L1

14	0	0	0	L2

15	1	1	0	L1

16	-1	1	1	L2

Prediction Variance Profile

11.24S.	An experimenter wishes to run a three-component mixture experiment.  The constraints in the components proportions are as follows:

(a)	Set up an experiment to fit a quadratic mixture model.  Use n = 14 runs, with 4 replicates.  Use the D-criteria.

| Std | x1 | x2 | x3 |
| --- | --- | --- | --- |
| 1 | 0.2 | 0.3 | 0.5 |
| 2 | 0.3 | 0.3 | 0.4 |
| 3 | 0.3 | 0.15 | 0.55 |
| 4 | 0.2 | 0.1 | 0.7 |
| 5 | 0.4 | 0.2 | 0.4 |
| 6 | 0.4 | 0.1 | 0.5 |
| 7 | 0.2 | 0.2 | 0.6 |
| 8 | 0.275 | 0.25 | 0.475 |
| 9 | 0.35 | 0.175 | 0.475 |
| 10 | 0.3 | 0.1 | 0.6 |
| 11 | 0.2 | 0.3 | 0.5 |
| 12 | 0.3 | 0.3 | 0.4 |
| 13 | 0.2 | 0.1 | 0.7 |
| 14 | 0.4 | 0.1 | 0.5 |

(b)	Draw the experimental design region.

(c)	Set up an experiment to fit a quadratic mixture model with n = 12 runs, assuming that three of these runs are replicated.  Use the D-criterion.

| Std | x1 | x2 | x3 |
| --- | --- | --- | --- |
| 1 | 0.3 | 0.15 | 0.55 |
| 2 | 0.2 | 0.3 | 0.5 |
| 3 | 0.3 | 0.3 | 0.4 |
| 4 | 0.2 | 0.1 | 0.7 |
| 5 | 0.4 | 0.2 | 0.4 |
| 6 | 0.4 | 0.1 | 0.5 |
| 7 | 0.2 | 0.2 | 0.6 |
| 8 | 0.275 | 0.25 | 0.475 |
| 9 | 0.35 | 0.175 | 0.475 |
| 10 | 0.2 | 0.1 | 0.7 |
| 11 | 0.4 | 0.1 | 0.5 |
| 12 | 0.4 | 0.2 | 0.4 |

(d)	Comment on the two designs you have found.

The design points are the same for both designs except that the edge center on the x1-x3 edge is not included in the second design.  None of the replicates for either design are in the center of the experimental region.  The experimental runs are fairly uniformly spaced in the design region.

11.25.	Myers, Montgomery and Anderson-Cook (2009) describe a gasoline blending experiment involving three mixture components.  There are no constraints on the mixture proportions, and the following 10 run design is used.

| Design Point | x1 | x2 | x3 | y(mpg) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 24.5, 25.1 |
| 2 | 0 | 1 | 0 | 24.8, 23.9 |
| 3 | 0 | 0 | 1 | 22.7, 23.6 |
| 4 | ½ | ½ | 0 | 25.1 |
| 5 | ½ | 0 | ½ | 24.3 |
| 6 | 0 | ½ | ½ | 23.5 |
| 7 | 1/3 | 1/3 | 1/3 | 24.8, 24.1 |
| 8 | 2/3 | 1/6 | 1/6 | 24.2 |
| 9 | 1/6 | 2/3 | 1/6 | 23.9 |
| 10 | 1/6 | 1/6 | 2/3 | 23.7 |

(a)	What type of design did the experimenters use?

A simplex centroid design was used.

(b)	Fit a quadratic mixture model to the data.  Is this model adequate?

Design Expert Output

Response:	y

ANOVA for Mixture Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	4.22	5	0.84	3.90	0.0435	significant

Linear Mixture	3.92	2	1.96	9.06	0.0088

AB	0.15	1	0.15	0.69	0.4289

AC	0.081	1	0.081	0.38	0.5569

BC	0.077	1	0.077	0.36	0.5664

Residual	1.73	8	0.22

Lack of Fit	0.50	4	0.12	0.40	0.8003	not significant

Pure Error	1.24	4	0.31

Cor Total	5.95	13

The Model F-value of 3.90 implies the model is significant.  There is only

a 4.35% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	0.47		R-Squared	0.7091

Mean	24.16		Adj R-Squared	0.5274

C.V.	1.93		Pred R-Squared	0.1144

PRESS	5.27		Adeq Precision	5.674

Coefficient		Standard	95% CI	95% CI

Component	Estimate	DF	Error	Low	High

A-x1	24.74	1	0.32	24.00	25.49

B-x2	24.31	1	0.32	23.57	25.05

C-x3	23.18	1	0.32	22.43	23.92

AB	1.51	1	1.82	-2.68	5.70

AC	1.11	1	1.82	-3.08	5.30

BC	-1.09	1	1.82	-5.28	3.10

Final Equation in Terms of Pseudo Components:

y	 =

+24.74	  * A

+24.31	  * B

+23.18	  * C

+1.51	  * A * B

+1.11	  * A * C

-1.09	  * B * C

Final Equation in Terms of Real Components:

y	 =

+24.74432	  * x1

+24.31098	  * x2

+23.17765	  * x3

+1.51364	  * x1 * x2

+1.11364	  * x1 * x3

-1.08636	  * x2 * x3

The quadratic terms appear to be insignificant.  The analysis below is for the linear mixture model:

Design Expert Output

Response:	y

ANOVA for Mixture Quadratic Model

Analysis of variance table [Partial sum of squares]

Sum of		Mean	F

Source	Squares	DF	Square	Value	Prob > F

Model	3.92	2	1.96	10.64	0.0027	significant

Linear Mixture	3.92	2	1.96	10.64	0.0027

Residual	2.03	11	0.18

Lack of Fit	0.79	7	0.11	0.37	0.8825	not significant

Pure Error	1.24	4	0.31

Cor Total	5.95	13

The Model F-value of 10.64 implies the model is significant.  There is only

a 0.27% chance that a "Model F-Value" this large could occur due to noise.

Std. Dev.	0.43		R-Squared	0.6591

Mean	24.16		Adj R-Squared	0.5972

C.V.	1.78		Pred R-Squared	0.3926

PRESS	3.62		Adeq Precision	8.751

Coefficient		Standard	95% CI	95% CI

Component	Estimate	DF	Error	Low	High

A-x1	24.93	1	0.25	24.38	25.48

B-x2	24.35	1	0.25	23.80	24.90

C-x3	23.19	1	0.25	22.64	23.74

Adjusted	Adjusted	Approx t for H0

Component	Effect	DF	Std Error	Effect=0	Prob > |t|

A-x1	1.16	1	0.33	3.49	0.0051

B-x2	0.29	1	0.33	0.87	0.4021

C-x3	-1.45	1	0.33	-4.36	0.0011

Final Equation in Terms of Pseudo Components:

y	 =

+24.93	  * A

+24.35	  * B

+23.19	  * C

Final Equation in Terms of Real Components:

y	 =

+24.93048	  * x1

+24.35048	  * x2

+23.19048	  * x3

(c)	Plot the response surface contours.  What blend would you recommend to maximize the MPG?

To maximize the miles per gallon, the recommended blend is x1 = 1, x2 = 0, and x3 = 0.

11.26.	Table P11.9 shows a six-variable RSM design from Jones and Nachtsheim (2011b).  Analyze the response data from this experiment.

Table P11.9 – The Design for Problem 11.26

| Run(i) | xi.1 | xi.2 | xi.3 | xi.4 | xi.5 | xi.6 | yi |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | -1 | -1 | -1 | -1 | 21.04 |
| 2 | 0 | -1 | 1 | 1 | 1 | 1 | 10.48 |
| 3 | 1 | 0 | -1 | 1 | 1 | -1 | 17.89 |
| 4 | -1 | 0 | 1 | -1 | -1 | 1 | 10.07 |
| 5 | -1 | -1 | 0 | 1 | -1 | -1 | 7.74 |
| 6 | 1 | 1 | 0 | -1 | 1 | 1 | 21.01 |
| 7 | -1 | 1 | 1 | 0 | 1 | -1 | 16.53 |
| 8 | 1 | -1 | -1 | 0 | -1 | 1 | 20.38 |
| 9 | 1 | -1 | 1 | -1 | 0 | -1 | 8.62 |
| 10 | -1 | 1 | -1 | 1 | 0 | 1 | 7.8 |
| 11 | 1 | 1 | 1 | 1 | -1 | 0 | 23.56 |
| 12 | -1 | -1 | -1 | -1 | 1 | 0 | 15.24 |
| 13 | 0 | 0 | 0 | 0 | 0 | 0 | 19.91 |

Design Expert Output

Response	1	Yi
	        ANOVA for Response Surface Reduced 2FI Model
	Analysis of variance table [Partial sum of squares - Type III]
		Sum of		Mean	F	p-value
	Source	Squares	df	Square	Value	Prob > F

Model	366.59	7	52.37	8.46	0.0159	significant

A-Xi.1	116.14	1	116.14	18.77	0.0075
	  B-Xi.2	75.52	1	75.52	12.20	0.0174
	  C-Xi.3	17.13	1	17.13	2.77	0.1570
	  E-Xi.5	0.27	1	0.27	0.043	0.8431
	  AB	84.75	1	84.75	13.70	0.0140
	  AC	64.42	1	64.42	10.41	0.0233
	  AE	109.00	1	109.00	17.61	0.0085
	Residual	30.94	5	6.19
	Cor Total	397.53	12

Std. Dev.	2.49		R-Squared		0.9222
	Mean	15.41		Adj R-Squared	0.8132
	C.V. %	16.15		Pred R-Squared	0.5242

PRESS	189.14		Adeq Precision	8.627

Coefficient		Standard	95% CI	95% CI
	Factor	Estimate	df	Error	Low	High	VIF

Intercept	15.41	1	0.69	13.63	17.18

A-Xi.1	3.41	1	0.79	1.39	5.43	1.00
	 B-Xi.2	2.75	1	0.79	0.73	4.77	1.00
	 C-Xi.3	-1.31	1	0.79	-3.33	0.71	1.00
	 E-Xi.5	-0.16	1	0.79	-2.19	1.86	1.00
	 AB	3.57	1	0.96	1.09	6.04	1.20
	 AC	-3.11	1	0.96	-5.59	-0.63	1.20
	 AE	-4.04	1	0.96	-6.52	-1.57	1.20

Final Equation in Terms of Actual Factors:

		 Yi	 =

+15.40538

+3.40800	  * Xi.1
		+2.74800	  * Xi.2
		-1.30900	  * Xi.3
		-0.16400	  * Xi.5
		+3.56550	  * Xi.1 * Xi.2
		-3.10850	  * Xi.1 * Xi.3
		-4.04350	  * Xi.1 * Xi.5

The main effects A and B are significant as well as the AB, AC and AE two-factor interactions.  Factors C and E are kept in the model to maintain hierarchy.   The normal probability plot of residuals shows a possible outlier in the upper right corner.

11.27.	An article in Quality Progress (“For Starbucks, It’s in the Bag,” March 2011, pp. 18-23) describes using a central composite design to improve the packaging of one-pound coffee.  The objective is to produce an airtight seal that is easy to open without damaging the top of the coffee bag.  The experimenters studied the three factors – x1 = plastic viscosity (300-400 centipoise), x2 = clamp pressure (170-190 psi), and x3 = plate gap (-3, +3 mm) and two responses – y1 = tear and y2 = leakage.  The design is shown in Table P11.10.  The tear response was measured on a scale from 0 to 9 (good to bad) and the leakage was proportion failing.  Each run used a sample of 20 bags for response measurement.

Table P11.10 – The Coffee Bag Experiment in Problem 11.27

| Run | Viscosity | Pressure | Plate gap | Tear | Leakage |
| --- | --- | --- | --- | --- | --- |
| Center | 350 | 180 | 0 | 0 | 0.15 |
| Axial | 350 | 170 | 0 | 0 | 0.5 |
| Factorial | 319 | 186 | 1.8 | 0.45 | 0.15 |
| Factorial | 380 | 174 | 1.8 | 0.85 | 0.05 |
| Center | 350 | 180 | 0 | 0.35 | 0.15 |
| Axial | 300 | 180 | 0 | 0.3 | 0.45 |
| Axial | 400 | 180 | 0 | 0.7 | 0.25 |
| Axial | 350 | 190 | 0 | 1.9 | 0 |
| Center | 350 | 180 | 0 | 0.25 | 0.05 |
| Factorial | 319 | 186 | -1.8 | 0.1 | 0.35 |
| Factorial | 380 | 186 | -1.8 | 0.15 | 0.4 |
| Axial | 350 | 180 | 3 | 3.9 | 0 |
| Factorial | 380 | 174 | -1.8 | 0 | 0.45 |
| Center | 350 | 180 | 0 | 0.55 | 0.2 |
| Axial | 350 | 180 | -3 | 0 | 1 |
| Factorial | 319 | 174 | -1.8 | 0.05 | 0.2 |
| Factorial | 319 | 174 | 1.8 | 0.4 | 0.25 |
| Factorial | 380 | 186 | 1.8 | 4.3 | 0.05 |
| Center | 350 | 180 | 0 | 0 | 0 |

(a)	Build a second-order model for the tear response.

Design Expert Output

Response	1	Tear
	        ANOVA for Response Surface Reduced Cubic Model
	Analysis of variance table [Partial sum of squares - Type III]
			Sum of		Mean	F	p-value
	Source		Squares	df	Square	Value	Prob > F

Model		26.40	8	3.30	13.72	0.0002	significant

A-Viscosity	1.81	1	1.81	7.52	0.0207
	  B-Pressure	3.48	1	3.48	14.47	0.0035
	  C-Plate Gap	11.00	1	11.00	45.73	< 0.0001
	  AB		1.53	1	1.53	6.36	0.0303
	  AC		2.31	1	2.31	9.60	0.0113
	  BC		1.36	1	1.36	5.66	0.0387
	  C2		3.54	1	3.54	14.72	0.0033
	  ABC		1.36	1	1.36	5.66	0.0387
	Residual		2.41	10	0.24
	Lack of Fit		2.18	6	0.36	6.53	0.0454	significant
	Pure Error		0.22	4	0.056
	Cor Total		28.81	18

Std. Dev.	0.49		R-Squared	0.9165
	Mean	0.75		Adj R-Squared	0.8497
	C.V. %	65.41		Pred R-Squared	0.4004

PRESS	17.27		Adeq Precision	14.560

Coefficient		Standard	95% CI	95% CI
	Factor		Estimate	df	Error	Low	High	VIF

Intercept		0.39	1	0.15	0.065	0.72

A-Viscosity	0.36	1	0.13	0.068	0.66	1.00
	 B-Pressure	0.50	1	0.13	0.21	0.80	1.00
	 C-Plate Gap	0.90	1	0.13	0.60	1.19	1.00
	 AB		0.44	1	0.17	0.051	0.82	1.00
	 AC		0.54	1	0.17	0.15	0.92	1.00
	 BC		0.41	1	0.17	0.026	0.80	1.00
	 C2		0.50	1	0.13	0.21	0.79	1.00
	 ABC		0.41	1	0.17	0.026	0.80	1.00

Final Equation in Terms of Actual Factors:

		 Tear	 =

+131.47081

-0.41839	  	* Viscosity
		-0.75140		* Pressure
		+68.98264	 	* Plate Gap
		+2.39071E-003	* Viscosity * Pressure
		-0.21562	  	* Viscosity * Plate Gap
		-0.39948	  	* Pressure * Plate Gap
		+0.15424	  	* Plate Gap2
		+1.25228E-003	* Viscosity * Pressure * Plate Gap

The second-order model for the tear response includes all three main effects, all two-factor interactions, the three-factor interaction and B2.

(b)	Build a second-order model for the leakage response.

Design Expert Output

Response	2	Leakage
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
			Sum of		Mean	F	p-value
	Source		Squares	df	Square	Value	Prob > F

Model		0.62	2	0.31	10.94	0.0010	significant

C-Plate Gap	0.49	1	0.49	17.13	0.0008
	  C2		0.14	1	0.14	4.75	0.0445
	Residual		0.46	16	0.028
	Lack of Fit		0.43	12	0.036	5.30	0.0603	not significant
	Pure Error		0.027	4	6.750E-003
	Cor Total		1.08	18

Std. Dev.	0.17		R-Squared		0.5776
	Mean	0.24		Adj R-Squared	0.5248
	C.V. %	68.98		Pred R-Squared	0.1949

PRESS	0.87		Adeq Precision	10.222

Coefficient		Standard		95% CI	95% CI
	Factor		Estimate	df	Error	Low		High	VIF

Intercept		0.17	1	0.050	0.068		0.28

C-Plate Gap	-0.19	1	0.046	-0.29		-0.092	1.00
	 C2		0.098	1	0.045	2.704E-003	0.19	1.00

Final Equation in Terms of Actual Factors:

		 Leakage	 =

+0.17449

-0.10503	  * Plate Gap
		+0.030162	  * Plate Gap2

For the Leakage response, only variable C – Plate Gap and C2 are important.

(c)	Analyze the residuals for both models.  Do transformations seem necessary for either response?   If so, refit the models in the transformed metric.

For the Tear response there is a little bit of non-linearity in the C response.

For the Tear response, the residuals look fine.  No concerns with model adequacy.

(d)	Construct response surface plots and contour plots for both responses.  Provide interpretations for the fitted surfaces.

For the Tear Response:

A and B with C=0

A and B with C=1

For the Leakage Response, factors A and C, with B=0

(e)	What conditions would you recommend for process optimization to minimize leakage and keep tear below 0.75.

Recommend running the process with C<0.  When C is >0 there is very little range where the Tear response is less than 0.75.  The graph below shows the A and B ranges when C=0.

11.28.	Box and Liu (1999) describe an experiment flying paper helicopters where the objective is to maximize flight time.  They used a central composite design shown in Table P11.11.  Each run involved a single helicopter made to the following specifications:  x1 = wing area (in2), -1 = 11.80 and +1 = 13.00; x2 = wing-length to width ratio, -1 = 2.25 and +1 = 2.78; x3 = base width (in), -1 = 1.00 and +1 = 1.50; and x4 = base length (in), -1 = 1.50 and +1 = 2.50.  Each helicopter was flown four times and the average flight time and the standard deviation of flight time was recorded.

Table P11.11- The Paper Helicopter Experiment

| Std Ord | Run Ord | Wing Area | Wing Ratio | Base Width | Base Length | Avg Flight Time | St Dev Flight Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 9 | -1 | -1 | -1 | -1 | 3.67 | 0.052 |
| 2 | 21 | 1 | -1 | -1 | -1 | 3.69 | 0.052 |
| 3 | 14 | -1 | 1 | -1 | -1 | 3.74 | 0.055 |
| 4 | 4 | 1 | 1 | -1 | -1 | 3.7 | 0.062 |
| 5 | 2 | -1 | -1 | 1 | -1 | 3.72 | 0.052 |
| 6 | 19 | 1 | -1 | 1 | -1 | 3.55 | 0.065 |
| 7 | 22 | -1 | 1 | 1 | -1 | 3.97 | 0.052 |
| 8 | 25 | 1 | 1 | 1 | -1 | 3.77 | 0.098 |
| 9 | 27 | -1 | -1 | -1 | 1 | 3.5 | 0.079 |
| 10 | 13 | 1 | -1 | -1 | 1 | 3.73 | 0.072 |
| 11 | 20 | -1 | 1 | -1 | 1 | 3.58 | 0.083 |
| 12 | 6 | 1 | 1 | -1 | 1 | 3.63 | 0.132 |
| 13 | 12 | -1 | -1 | 1 | 1 | 3.44 | 0.058 |
| 14 | 17 | 1 | -1 | 1 | 1 | 3.55 | 0.049 |
| 15 | 26 | -1 | 1 | 1 | 1 | 3.7 | 0.081 |
| 16 | 1 | 1 | 1 | 1 | 1 | 3.62 | 0.051 |
| 17 | 8 | -2 | 0 | 0 | 0 | 3.61 | 0.129 |
| 18 | 15 | 2 | 0 | 0 | 0 | 3.64 | 0.085 |
| 19 | 7 | 0 | -2 | 0 | 0 | 3.55 | 0.1 |
| 20 | 5 | 0 | 2 | 0 | 0 | 3.73 | 0.063 |
| 21 | 29 | 0 | 0 | -2 | 0 | 3.61 | 0.051 |
| 22 | 28 | 0 | 0 | 2 | 0 | 3.6 | 0.095 |
| 23 | 16 | 0 | 0 | 0 | -2 | 3.8 | 0.049 |
| 24 | 18 | 0 | 0 | 0 | 2 | 3.6 | 0.055 |
| 25 | 24 | 0 | 0 | 0 | 0 | 3.77 | 0.032 |
| 26 | 10 | 0 | 0 | 0 | 0 | 3.75 | 0.055 |
| 27 | 23 | 0 | 0 | 0 | 0 | 3.7 | 0.072 |
| 28 | 11 | 0 | 0 | 0 | 0 | 3.68 | 0.055 |
| 29 | 3 | 0 | 0 | 0 | 0 | 3.69 | 0.078 |
| 30 | 30 | 0 | 0 | 0 | 0 | 3.66 | 0.058 |

(a)	Fit a second-order model to the average flight time response.

The flight time response is driven by B and D, several two-factor interactions and A2 and C2 terms.  A and C are in the model to maintain hierarchy.

Design Expert Output

Response	1	Avg Flt Time
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
			Sum of			Mean		F	p-value
	Source		Squares		df	Square		Value	Prob > F

Model		0.28		11	0.025		15.78	< 0.0001	significant

A-Wing Area	1.667E-005	1	1.667E-005	0.010	0.9198
	  B-Length/Width	0.062		1	0.062		38.83	< 0.0001
	  C-Base Width	1.500E-004	1	1.500E-004	0.094	0.7628
	  D-Base Length	0.089		1	0.089		55.61	< 0.0001
	  AB		0.013		1	0.013		8.28	0.0100
	  AC		0.023		1	0.023		14.09	0.0015
	  AD		0.031		1	0.031		19.17	0.0004
	  BC		0.034		1	0.034		21.43	0.0002
	  CD		7.225E-003	1	7.225E-003	4.52	0.0475
	  A2		7.511E-003	1	7.511E-003	4.70	0.0438
	  C2		0.013		1	0.013		8.04	0.0110
	Residual		0.029		18	1.597E-003
	Lack of Fit		0.020		13	1.513E-003	0.83	0.6382	not significant
	Pure Error		9.083E-003	5	1.817E-003
	Cor Total		0.31		29

Std. Dev.	0.040		R-Squared		0.9060
	Mean	3.66		Adj R-Squared	0.8486
	C.V. %	1.09		Pred R-Squared	0.7581

PRESS	0.074		Adeq Precision	19.913

Coefficient		Standard		95% CI	95% CI
	Factor		Estimate		df	Error		Low	High		VIF

Intercept		3.70		1	0.012		3.67	3.72

A-Wing Area	-8.333E-004	1	8.158E-003	-0.018	0.016		1.00
	 B-Length/Width 	0.051		1	8.158E-003	0.034	0.068		1.00
	 C-Base Width	2.500E-003	1	8.158E-003	-0.015	0.020		1.00
	 D-Base Length	-0.061		1	8.158E-003	-0.078	-0.044		1.00
	 AB		-0.029		1	9.991E-003	-0.050	-7.759E-003	1.00
	 AC		-0.037		1	9.991E-003	-0.058	-0.017		1.00
	 AD		0.044		1	9.991E-003	0.023	0.065		1.00
	 BC		0.046		1	9.991E-003	0.025	0.067		1.00
	 CD		-0.021		1	9.991E-003	-0.042	-2.590E-004	1.00
	 A2		-0.016		1	7.493E-003	-0.032	-5.068E-004	1.01
	 C2		-0.021		1	7.493E-003	-0.037	-5.507E-003	1.01

Final Equation in Terms of Actual Factors:

		 Avg Flt Time	 =

+3.70

-8.333E-004	* A
		+0.051	  	* B
		+2.500E-003	* C
		-0.061	  	* D
		-0.029	  	* A * B
		-0.037	  	* A * C
		+0.044	  	* A * D
		+0.046	  	* B * C
		-0.021	  	* C * D
		-0.016	  	* A2
		-0.021	  	* C2

	 Final Equation in Terms of Actual Factors:

		 Avg Flt Time	 =
		-8.13881
		+1.59365	  * Wing Area
		+1.56132	  * Length/Width Ratio
		+2.54425	  * Base Width
		-1.71750	  * Base Length
		-0.18082	  * Wing Area * Length/Width Ratio
		-0.25000	  * Wing Area * Base Width
		+0.14583	  * Wing Area * Base Length
		+0.69811	  * Length/Width Ratio * Base Width
		-0.17000	  * Base Width * Base Length
		-0.045139	  * Wing Area2
		-0.34000	  * Base Width2

(b)	Fit a second-order model to the standard deviation of flight time response.

The standard deviation response is driven by the CD interaction and A2.  A, C and D are in the model to maintain hierarchy.

Design Expert Output

Response	2	Stdev Flt Time
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
			Sum of			Mean		F	p-value
	Source		Squares		df	Square		Value	Prob > F

Model		5.562E-003	5	1.112E-003	2.57	0.0533	not significant

A-Wing Area	1.504E-005	1	1.504E-005	0.035	0.8536
	  C-Base Width	2.042E-006	1	2.042E-006	4.72E-003	0.9458
	  D-Base Length	6.934E-004	1	6.934E-004	1.60	0.2176
	  CD		1.871E-003	1	1.871E-003	4.32	0.0484
	  A2		2.981E-003	1	2.981E-003	6.89	0.0148
	Residual		0.010		24	4.325E-004
	Lack of Fit		9.091E-003	19	4.785E-004	1.86	0.2556	not significant
	Pure Error		1.289E-003	5	2.579E-004
	Cor Total		0.016		29

Std. Dev.	0.021		R-Squared		0.3489
	Mean	0.069		Adj R-Squared	0.2132
	C.V. %	30.14		Pred R-Squared	-0.1717

PRESS	0.019		Adeq Precision	5.702

Coefficient		Standard	95% CI		95% CI
	Factor		Estimate		df	Error	Low		High		VIF

Intercept		0.061		1	4.90E-003	0.051		0.071

A-Wing Area	-7.917E-004	1	4.24E-003	-9.55E-003	7.970E-003	1.00
	 C-Base Width	2.917E-004	1	4.24E-003	-8.47E-003	9.053E-003	1.00
	 D-Base Length	5.375E-003	1	4.24E-003	-3.38E-003	0.014		1.00
	 CD		-0.011		1	5.19E-003	-0.022		-8.191E-005	1.00
	 A2		0.010		1	3.87E-003	2.175E-003	0.018		1.00

Final Equation in Terms of Coded Factors:

		 Stdev Flt Time	 =

+0.061

-7.917E-004	  * A
		+2.917E-004	  * C
		+5.375E-003	  * D
		-0.011	  	  * C * D
		+0.010	  	  * A2

Final Equation in Terms of Actual Factors:

		 Stdev Flt Time	 =
		+4.18328
		-0.70217	  * Wing Area
		+0.17417	  * Base Width
		+0.11887	  * Base Length
		-0.086500	  * Base Width * Base Length
		+0.028260	  * Wing Area2

(c)	Analyze the residuals for both models from parts (a) and (b).  Are transformations on the response(s) necessary?  If so, fit the appropriate models.

For the average flight time response the residuals look fine.  The residual by factor are also okay.

For the standard deviation of flight time response the plot of residuals vs. predicted shows an inequality of variance.  Recommend doing a Log transformation.

Design Expert Output

Response	2	Stdev Flt Time
	Transform:	Base 10 Log	Constant:	0
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
			Sum of			Mean		F		p-value
	Source		Squares		df	Square		Value		Prob > F

Model		0.18		5	0.035		2.34		0.0725	not sig

A-Wing Area	1.089E-004	1	1.089E-004	7.212E-003	0.9330
	  C-Base Width	3.136E-004	1	3.136E-004	0.021		0.8866
	  D-Base Length	0.024		1	0.024		1.61		0.2167
	  CD		0.062		1	0.062		4.11		0.0539
	  A2		0.090		1	0.090		5.96		0.0224
	Residual		0.36		24	0.015
	Lack of Fit		0.27		19	0.014		0.77		0.6920	not sig
	Pure Error		0.092		5	0.018
	Cor Total		0.54		29

Std. Dev.	0.12		R-Squared	0.3278
	Mean	-1.18		Adj R-Squared	0.1878
	C.V. %	10.39		Pred R-Squared	-0.0669

PRESS	0.58		Adeq Precision	5.304

Coefficient		Standard	95% CI	95% CI
	Factor		Estimate		df	Error	Low	High	VIF

Intercept		-1.23		1	0.029	-1.29	-1.17

A-Wing Area	-2.130E-003	1	0.025	-0.054	0.050	1.00
	 C-Base Width	3.615E-003	1	0.025	-0.048	0.055	1.00
	 D-Base Length	0.032		1	0.025	-0.020	0.084	1.00
	 CD		-0.062		1	0.031	-0.13	1.13E-003	1.00
	 A2		0.056		1	0.023	8.64E-003	0.10	1.00

Final Equation in Terms of Coded Factors:

		 Log10(Stdev Flt Time)	 =

-1.23

-2.130E-003	  * A
		+3.615E-003	  * C
		+0.032	 	  * D
		-0.062	  	  * C * D
		+0.056	  	  * A2

Final Equation in Terms of Actual Factors:

		 Log10(Stdev Flt Time)	 =
		+21.30149
		-3.85446	  * Wing Area
		+1.01078	  * Base Width
		+0.68634	  * Base Length
		-0.49816	  * Base Width * Base Length
		+0.15528	  * Wing Area2

The residual improve slightly with the transformed model.

(d)	What design would you recommend to maximize the flight time?

Run A at the low level, B at the high level, C at the high level and D at the low level.  This give a flight time of almost 4 seconds.

(e)	What design would you recommend to maximize the flight time while simultaneously minimizing the standard deviation of flight time?

Running A at -0.715, B at -2, C at -0.261 and D at -2, in coded terms.  Applying these values to the regression equations estimates the hang time of 3.97 seconds and the standard deviation at 0.051.

11.29.	The Paper Helicopter Experiment Revisited.  Reconsider the paper helicopter experiment in Problem 11.28.  This experiment was actually run in two blocks.  Block 1 consisted of the first 16 runs in Table P11.11 (standard order runs 1-16) and two center points (standard order runs 25 and 26).

(a)	Fit the main-effects plus two-factor interaction models to the block 1 data, using both responses.

For the average flight time:  The main effects of B and D are significant as well as the two-factor interactions of AB, AC, AD, BC.  A and C are added to maintain hierarchy.

Design Expert Output

ANOVA for selected factorial model
	Analysis of variance table [Partial sum of squares - Type III]

Sum of		Mean	F	p-value
	Source		Squares	df	Square	Value	Prob > F

Model		0.22	8	0.027	12.99	0.0008	significant
	  A-Wing Area	4.00E-004	1	4.00E-004	0.19	0.6738
	  B-Wing Ratio	0.046	1	0.046	22.04	0.0016
	  C-Base Width	4.00E-004	1	4.00E-004	0.19	0.6738
	  D-Base Length	0.070	1	0.070	33.49	0.0004
	  AB		0.013	1	0.013	6.31	0.0363
	  AC		0.022	1	0.022	10.73	0.0113
	  AD		0.031	1	0.031	14.61	0.0051
	  BC		0.034	1	0.034	16.32	0.0037
	Curvature		0.018	1	0.018	8.48	0.0195	significant
	Residual		0.017	8	2.09E-003
	Lack of Fit		0.017	7	2.36E-003	11.84	0.2202	not significant
	Pure Error		2.00E-004	1	2.00E-004
	Cor Total		0.25	17

Std. Dev.	0.062		R-Squared		0.8631
	Mean	3.67		Adj R-Squared	0.7414
	C.V. %	1.69		Pred R-Squared	0.5568

PRESS	0.11		Adeq Precision	10.899

Coefficient		Standard	95% CI	95% CI

Factor		Estimate	df	Error	Low	High	VIF
	 Intercept		3.67	1	0.015	3.64	3.70
	 A-Wing Area           -5.00E-003	1	0.015	-0.040	0.030	1.00
	 B-Wing Ratio	0.054	1	0.015	0.019	0.089	1.00
	 C-Base Width	5.00E-003	1	0.015	-0.030	0.040	1.00
	 D-Base Length	-0.066	1	0.015	-0.10	-0.031	1.00
	 AB		-0.029	1	0.015	-0.064	6.29E-003	1.00
	 AC		-0.037	1	0.015	-0.073       -2.46E-003	1.00
	 AD		0.044	1	0.015	8.71E-003	0.079	1.00
	 BC		0.046	1	0.015	0.011	0.081	1.00

Final Equation in Terms of Coded Factors:

Avg Flt Time	 =
		+3.67
		-5.00E-003  * A
		+0.054	  * B
		+5.00E-003 * C
		-0.066	  * D
		-0.029	  * A * B
		-0.037	  * A * C
		+0.044	  * A * D
		+0.046	  * B * C

For the Standard Deviation Flight Time response:  Only the CD interaction is significant.  C and D are added to maintain hierarchy.

Design Expert Output

ANOVA for selected factorial model
	Analysis of variance table [Partial sum of squares - Type III]

Sum of			Mean		F	p-value
	Source		Squares		df	Square		Value	Prob > F

Model		3.136E-003	3	1.045E-003	2.93	0.0735	not significant
	  C-Base Width	4.101E-004	1	4.101E-004	1.15	0.3034
	  D-Base Length	8.556E-004	1	8.556E-004	2.40	0.1456
	  CD		1.871E-003	1	1.871E-003	5.24	0.0395
	Curvature		1.095E-003	1	1.095E-003	3.07	0.1035	not significant
	Residual		4.642E-003	13	3.571E-004
	Lack of Fit		4.377E-003	12	3.648E-004	1.38	0.5889	not significant
	Pure Error		2.645E-004	1	2.645E-004
	Cor Total		8.872E-003	17

Std. Dev.	0.020		R-Squared	0.3535
	Mean	0.066		Adj R-Squared	0.2149
	C.V. %	30.88		Pred R-Squared	-0.0413

PRESS	9.239E-003	Adeq Precision	3.799

Coefficient		Standard		95% CI	95% CI

Factor		Estimate		df	Error		Low	High		VIF
	 Intercept		0.066		1	4.771E-003	0.055	0.076
	 C-Base Width	-5.063E-003	1	5.060E-003	-0.016	5.791E-003	1.00
	 D-Base Length	7.312E-003	1	5.060E-003              -3.54E-003	0.018		1.00
	 CD		-0.011		1	5.060E-003	-0.022	4.111E-005	1.00

Final Equation in Terms of Coded Factors:

Stdev Flt Time	 =
		+0.066
		-5.063E-003	  * C
		+7.312E-003	  * D
		-0.011	  	  * C * D

(b)	For the models in part (a) use the two center points to test for lack of fit.  Is there an indication that second-order terms are needed?

The ANOVA for Average Flight Time shows curvature to be significant.  The ANOVA for Standard Deviation Flight Time does not show curvature.

(c)	Now use the data from block 2 (standard order runs 17-24 and the remaining center points, standard order runs 27-30) to augment block 1 and fit second-order models to both responses.  Check the adequacy of the fit for both models.  Does blocking seem to have been important in this experiment?

Blocking has little effect on the models.

For the average flight time of the combined design:  The main effects of B and D are significant as well as the two-factor interactions of AB, AC, AD, BC and CD. The quadratic terms of A2, B2 and C2 are significant.  A and C are added to maintain hierarchy.

Design Expert Output

Response	1	Avg Flt Time
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
			Sum of		Mean		F	p-value
	Source		Squares	df	Square		Value	Prob > F

Block		1.68E-003	1	1.681E-003

Model		0.29	12	0.024		22.14	< 0.0001	significant
	    A-Wing Area	1.67E-005	1	1.667E-005	0.015	0.9027
	    B-Wing Ratio	0.062	1	0.062		57.40	< 0.0001
	    C-Base Width	1.50E-004	1	1.500E-004	0.14	0.7143
	    D-Base Length	0.089	1	0.089		82.21	< 0.0001
	    AB		0.013	1	0.013		12.24	0.0030
	    AC		0.022	1	0.022		20.83	0.0003
	    AD		0.031	1	0.031		28.35	< 0.0001
	    BC		0.034	1	0.034		31.68	< 0.0001
	    CD		7.22E-003	1	7.225E-003	6.69	0.0199
	    A2		0.011	1	0.011		10.26	0.0055
	    B2		7.33E-003	1	7.336E-003	6.79	0.0191
	    C2		0.017	1	0.017		16.00	0.0010
	Residual		0.017	16	1.080E-003
	Lack of Fit		0.016	12	1.351E-003	5.03	0.0659	not significant
	Pure Error		1.07E-003	4	2.688E-004
	Cor Total		0.31	29

Std. Dev.	0.033		R-Squared		0.9432
	Mean	3.66		Adj R-Squared	0.9006
	C.V. %	0.90		Pred R-Squared	0.7728

PRESS	0.069		Adeq Precision	22.417

Coefficient	Standard		95% CI	95% CI
	Factor		Estimate	df	Error		Low	High		VIF

Intercept		3.71	1	0.011	3	.69	3.74

Block 1		0.014	1
	 Block 2		-0.014

A-Wing Area            -8.33E-004	1	6.709E-003	-0.015	0.013		1.00
	 B-Wing Ratio	0.051	1	6.709E-003	0.037	0.065		1.00
	 C-Base Width	2.50E-003	1	6.709E-003	-0.012	0.017		1.00
	 D-Base Length	-0.061	1	6.709E-003	-0.075	-0.047		1.00
	 AB		-0.029	1	8.217E-003	-0.046	-0.011		1.00
	 AC		-0.037	1	8.217E-003	-0.055	-0.020		1.00
	 AD		0.044	1	8.217E-003	0.026	0.061		1.00
	 BC		0.046	1	8.217E-003	0.029	0.064		1.00
	 CD		-0.021	1	8.217E-003	-0.039	-3.830E-003	1.00
	 A2		-0.020	1	6.276E-003	-0.033	-6.800E-003	1.05
	 B2		-0.016	1	6.276E-003	-0.030	-3.050E-003	1.05
	 C2		-0.025	1	6.276E-003	-0.038	-0.012		1.05

Final Equation in Terms of Coded Factors:

		 Avg Flt Time	 =

+3.71

-8.333E-004	  * A
		+0.051	 	  * B
		+2.500E-003	  * C
		-0.061	 	  * D
		-0.029		  * A * B
		-0.037		  * A * C
		+0.044		  * A * D
		+0.046		  * B * C
		-0.021		  * C * D
		-0.020		  * A2
		-0.016	 	  * B2
		-0.025	  	  * C2

The residual analysis for the Average Flight Time response shows a little inequality of variance in Factor A.

For the Standard Deviation Flight Time response in the combined model:  Only the CD interaction and the A2 term are significant.  A, C and D are added to maintain hierarchy.

Design Expert Output

Response	2	Stdev Flt Time
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]	
			Sum of			Mean		F	p-value
	Source		Squares		df	Square		Value	Prob > F

Block		5.339E-004	1	5.339E-004

Model		5.890E-003	5	1.178E-003	2.85	0.0383	significant
	    A-Wing Area	1.504E-005	1	1.504E-005	0.036	0.8505
	    C-Base Width	2.042E-006	1	2.042E-006	4.93E-003	0.9446
	    D-Base Length	6.934E-004	1	6.934E-004	1.68	0.2084
	    CD		1.871E-003	1	1.871E-003	4.52	0.0445
	    A2		3.309E-003	1	3.309E-003	7.99	0.0095
	Residual		9.518E-003	23	4.138E-004
	Lack of Fit		8.889E-003	19	4.679E-004	2.97	0.1499	not significant
	Pure Error		6.292E-004	4	1.573E-004
	Cor Total		0.016		29

Std. Dev.	0.020		R-Squared		0.3822
	Mean	0.069		Adj R-Squared	0.2479
	C.V. %	29.48		Pred R-Squared	-0.3520

PRESS	0.021		Adeq Precision	6.331

Coefficient	Standard		95% CI		95% CI
	Factor		Estimate	  df	Error		Low		High		VIF

Intercept		0.061	   1	4.814E-003	0.052		0.071

Block 1		-5.50E-003   1
	 Block 2		5.50E-003

A-Wing Area	-7.92E-004   1	4.153E-003	-9.382E-003	7.799E-003	1.00
	 C-Base Width	2.92E-004	   1	4.153E-003	-8.299E-003	8.882E-003	1.00
	 D-Base Length	5.37E-003	   1	4.153E-003	-3.215E-003	0.014		1.00
	 CD		-0.011	   1	5.086E-003	-0.021		-2.917E-004	1.00
	 A2		0.011	   1	3.814E-003	2.895E-003	0.019		1.01

Final Equation in Terms of Coded Factors:

		 Stdev Flt Time	 =

+0.061

-7.917E-004	  * A
		+2.917E-004	  * C
		+5.375E-003	  * D
		-0.011	 	  * C * D
		+0.011		  * A2

The residual analysis of the Standard Deviation Flight Time response shows in equality of variance in the Residuals vs. Predicted graph and in Factor B.  Might want to try a transformation on the Standard Deviation Response.

(d)	What design would you recommend to maximize the flight time while simultaneously minimizing the standard deviation of flight time?

A = -0.71, B = 2, C = 0.05, D = -2, in coded variables gives a predicted Flight Time of 3.967 and a predicted Standard Deviation of Flight Time of 0.058.

11.30.	An article in the Journal of Chromatography A (“Optimization of the Capillary Electrophoresis Separation of Ranitidine and Related Compounds,” Vol. 766, pp. 245-254) describes an experiment to optimize the production of ranitidine, a compound that is the primary active ingredient of Zantac, a pharmaceutical product used to treat ulcers, gastroesophageal reflux disease (a condition in which backward flow of acid from the stomach causes heartburn and injury of the esophagus), and other conditions where the stomach produces too much acid, such as Zollinger-Ellison syndrome.  The authors used three factors (x1 = pH of the buffer solution, x2 = the electrophoresis voltage, and the concentration of one component of the buffer solution) in a central composite design.  The response is chromatographic exponential function (CEF), which should be minimized.  Table P11.12 shows the design.

P11.12 – The Ranitidine Separation Experiment

| Std Ord | X1 | X2 | X3 | CEF |
| --- | --- | --- | --- | --- |
| 1 | -1 | -1 | -1 | 17.3 |
| 2 | 1 | -1 | -1 | 45.5 |
| 3 | -1 | 1 | -1 | 10.3 |
| 4 | 1 | 1 | -1 | 11757.1 |
| 5 | -1 | -1 | 1 | 16.942 |
| 6 | 1 | -1 | 1 | 25.4 |
| 7 | -1 | 1 | 1 | 31697.2 |
| 8 | 1 | 1 | 1 | 12039.2 |
| 9 | -1.68 | 0 | 0 | 7.5 |
| 10 | 1.68 | 0 | 0 | 6.3 |
| 11 | 0 | -1.68 | 0 | 11.1 |
| 12 | 0 | 1.68 | 0 | 6.6664 |
| 13 | 0 | 0 | -1.68 | 16548.7 |
| 14 | 0 | 0 | 1.68 | 26351.8 |
| 15 | 0 | 0 | 0 | 9.9 |
| 16 | 0 | 0 | 0 | 9.6 |
| 17 | 0 | 0 | 0 | 8.9 |
| 18 | 0 | 0 | 0 | 8.8 |
| 19 | 0 | 0 | 0 | 8.013 |
| 20 | 0 | 0 | 0 | 8.059 |

(a)	Fit the second-order model to the CEF response.  Analyze the residuals from this model.  Does it seem that all model terms are necessary?

There are many terms with low F-Values.  The Adjusted R-Square is low relative to the R-Square indicating too many terms in the model.

Design Expert Output

Response	1	CEF
	        ANOVA for Response Surface Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
		Sum of			Mean		F		p-value
	Source	Squares		df	Square		Value		Prob > F

Model	1.480E+009	9	1.644E+008	5.66		0.0061	significant

A-pH	4.543E+006	1	4.543E+006	0.16		0.7008
	  B-voltage	2.247E+008	1	2.247E+008	7.73		0.0194
	  C-concen	1.718E+008	1	1.718E+008	5.91		0.0353
	  AB	7.896E+006	1	7.896E+006	0.27		0.6134
	  AC	1.234E+008	1	1.234E+008	4.25		0.0662
	  BC	1.279E+008	1	1.279E+008	4.40		0.0622
	  A2	2.637E+005	1	2.637E+005	9.080E-003	0.9260
	  B2	2.610E+005	1	2.610E+005	8.986E-003	0.9264
	  C2	7.990E+008	1	7.990E+008	27.51		0.0004
	Residual	2.905E+008	10	2.905E+007
	Lack of Fit	2.905E+008	5	5.809E+007	9.710E+007	< 0.0001	significant
	Pure Error	2.99		5	0.60
	Cor Total	1.770E+009	19

Std. Dev.	5389.41		R-Squared		0.8359
	Mean	4929.71		Adj R-Squared	0.6882
	C.V. %	109.32		Pred R-Squared	-0.3559

PRESS	2.40E+009		Adeq Precision	8.981

Coefficient	Standard	95% CI	95% CI
	Factor		Estimate	df	Error	Low	High	VIF

Intercept		29.47	1	2198.06	-4868.12	4927.06

A-pH		-576.75	1	1458.36	-3826.18	2672.69	1.00
	 B-voltage		4055.93	1	1458.36	806.49	7305.36	1.00
	 C-concentration	3546.59	1	1458.36	297.16	6796.03	1.00
	 AB		-993.48	1	1905.44	-5239.08	3252.11	1.00
	 AC		-3928.07	1	1905.44	-8173.66	317.53	1.00
	 BC		3998.68	1	1905.44	-246.91	8244.28	1.00
	 A2		-135.28	1	1419.68	-3298.51	3027.96	1.02
	 B2		-134.58	1	1419.68	-3297.81	3028.66	1.02
	 C2		7446.09	1	1419.68	4282.85	10609.33	1.02

Final Equation in Terms of Coded Factors:

		 CEF	 =

+29.47

-576.75	  * A
		+4055.93	  * B
		+3546.59	  * C
		-993.48	  * A * B
		-3928.07	  * A * C
		+3998.68	  * B * C
		-135.28	  * A2
		-134.58	  * B2
		+7446.09	  * C2

The Normal Plot of residuals is somewhat S-shaped.

(b)	Reduce the model from part (a) as necessary.  Did the model reduction improve the fit?

The Adjusted R-Square improved.  The only terms left in the model have significant F-Values.

Design Expert Output

Response	1	CEF
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
		Sum of			Mean		F	p-value
	Source	Squares		df	Square		Value	Prob > F

Model	1.471E+009	6	2.452E+008	10.67	0.0002	significant

A-pH	4.543E+006	1	4.543E+006	0.20	0.6640
	  B-voltage	2.247E+008	1	2.247E+008	9.77	0.0080
	  C-concen	1.718E+008	1	1.718E+008	7.47	0.0171
	  AC	1.234E+008	1	1.234E+008	5.37	0.0374
	  BC	1.279E+008	1	1.279E+008	5.56	0.0346
	  C2	8.190E+008	1	8.190E+008	35.63	< 0.0001
	Residual	2.988E+008	13	2.299E+007
	Lack of Fit	2.988E+008	8	3.735E+007          6.244E+007	< 0.0001	significant
	Pure Error	2.99		5	0.60
	Cor Total	1.770E+009	19

Std. Dev.	4794.47		R-Squared		0.8312
	Mean	4929.71		Adj R-Squared	0.7533
	C.V. %	97.26		Pred R-Squared	0.3793

PRESS	1.099E+009	Adeq Precision	11.957

(c)	Does transformation of the CEF response seem like a useful idea?  What aspect of either the data or the residuals suggests that transformation would be helpful?

The Normal Plot of Residuals still shows an S-shaped pattern in the reduced model.  Suggest an inverse square root transformation.

(d)	Fit a second-order model to the transformed CEF response.  Analyze the residuals from this model.  Does it seem that all model terms are necessary?  What would you choose as the final model?

Like the first model, there are too many terms.  Many with low F-Values that could be deleted from the model.

Design Expert Output

Response	1	CEF
	Transform:	Inverse Sqrt	Constant:	0
	        ANOVA for Response Surface Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
		Sum of			Mean		F	p-value
	Source	Squares		df	Square		Value	Prob > F

Model	0.33		9	0.036		4.59	0.0130	significant

A-pH	0.011		1	0.011		1.33	0.2757
	  B-voltage	8.864E-003	1	8.864E-003	1.12	0.3155
	  C-concen	4.802E-003	1	4.802E-003	0.60	0.4547
	  AB	3.287E-003	1	3.287E-003	0.41	0.5344
	  AC	0.016		1	0.016		1.97	0.1910
	  BC	0.016		1	0.016		2.03	0.1850
	  A2	1.819E-004	1	1.819E-004	0.023	0.8827
	  B2	4.168E-003	1	4.168E-003	0.52	0.4853
	  C2	0.27		1	0.27		33.61	0.0002
	Residual	0.079		10	7.939E-003
	Lack of Fit	0.078		5	0.016		73.24	0.0001	significant
	Pure Error	1.069E-003	5	2.139E-004
	Cor Total	0.41	19

Std. Dev.	0.089		R-Squared		0.8050
	Mean	0.23		Adj R-Squared	0.6295
	C.V. %	38.33		Pred R-Squared	-0.4939

PRESS	0.61		Adeq Precision	7.192

The model residuals have improved.

The reduced model includes only factor C and C2.

Design Expert Output

Response	1	CEF
	Transform:	Inverse Sqrt	Constant:	0
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
		Sum of			Mean		F	p-value
	Source	Squares		df	Square		Value	Prob > F

Model	0.27		2	0.13		16.57	0.0001	significant

C-conce	4.802E-003	1	4.802E-003	0.59	0.4524
	  C2	0.26		1	0.26		32.56	< 0.0001
	Residual	0.14		17	8.119E-003
	Lack of Fit	0.14		12	0.011		53.36	0.0002	significant
	Pure Error	1.069E-003	5	2.139E-004
	Cor Total	0.41		19

Std. Dev.	0.090		R-Squared		0.6610
	Mean	0.23		Adj R-Squared	0.6211
	C.V. %	38.76		Pred R-Squared	0.4588

PRESS	0.22		Adeq Precision	11.782

(e)	Suppose you had some information that suggests that the separation process malfunctioned during run 7.  Delete this run and analyze the data from this experiment again.

Deleting run 7 does not have much impact on the experiment.

Design Expert Output

Response	1	CEF
	Transform:	Inverse Sqrt	Constant:	0
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
		Sum of			Mean		F	p-value
	Source	Squares		df	Square		Value	Prob > F

Model	0.25		2	0.12		18.51	< 0.0001	significant

C-conce	2.928E-004	1	2.928E-004	0.044	0.8366
	  C2	0.25		1	0.25		37.01	< 0.0001
	Residual	0.11		16	6.658E-003
	Lack of Fit	0.11		11	9.587E-003	44.83	0.0003	significant
	Pure Error	1.069E-003	5	2.139E-004
	Cor Total	0.35		18

Std. Dev.	0.082		R-Squared		0.6982
	Mean	0.24		Adj R-Squared	0.6605
	C.V. %	33.39		Pred R-Squared	0.5392

PRESS	0.16		Adeq Precision	11.599

(f)	What conditions would you recommend to minimize CEF?

Run C at the zero level minimizes CEF.

11.31.	An article in the Electronic Journal of Biotechnology (“Optimization of Medium Composition for Transglutaminase Production by a Brazilian Soil Streptomyces sp,” available at http://www.ejbiotechnology.info/content/vol10/issue4/full/10.index.html ) describes the use of designed experiments to improve the medium for cells used in a new microbial source of transglutaminase (MTGase), an enzyme that catalyzes an acyl transfer reaction using peptide-bond glutamine residues as acyl donors and some primary amines as acceptors.  Reactions catalyzed by MTGase can be used in food processing.  The article describes two phases of experimentation – screening with a fractional factorial and optimization.  We will use only the optimization experiment.  The design was a central composite design in four factors – x1 = KH2PO4, x2 = MgSO4-7H2O, x3 = soybean flower, and x4 = peptone.  MTGase activity is the response, which should be maximized.  Table P11.13 contains the design and the response data.

Table P11.13 – The MTGase Optimization Experiment

| Std Ord | X1 | X2 | X3 | X4 | MTGase Activity |
| --- | --- | --- | --- | --- | --- |
| 1 | -1 | -1 | -1 | -1 | 0.87 |
| 2 | 1 | -1 | -1 | -1 | 0.74 |
| 3 | -1 | 1 | -1 | -1 | 0.51 |
| 4 | 1 | 1 | -1 | -1 | 0.99 |
| 5 | -1 | -1 | 1 | -1 | 0.67 |
| 6 | 1 | -1 | 1 | -1 | 0.72 |
| 7 | -1 | 1 | 1 | -1 | 0.81 |
| 8 | 1 | 1 | 1 | -1 | 1.01 |
| 9 | -1 | -1 | -1 | 1 | 1.33 |
| 10 | 1 | -1 | -1 | 1 | 0.7 |
| 11 | -1 | 1 | -1 | 1 | 0.82 |
| 12 | 1 | 1 | -1 | 1 | 0.78 |
| 13 | -1 | -1 | 1 | 1 | 0.36 |
| 14 | 1 | -1 | 1 | 1 | 0.23 |
| 15 | -1 | 1 | 1 | 1 | 0.21 |
| 16 | 1 | 1 | 1 | 1 | 0.44 |
| 17 | -2 | 0 | 0 | 0 | 0.56 |
| 18 | 2 | 0 | 0 | 0 | 0.49 |
| 19 | 0 | -2 | 0 | 0 | 0.57 |
| 20 | 0 | 2 | 0 | 0 | 0.81 |
| 21 | 0 | 0 | -2 | 0 | 0.9 |
| 22 | 0 | 0 | 2 | 0 | 0.65 |
| 23 | 0 | 0 | 0 | -2 | 0.91 |
| 24 | 0 | 0 | 0 | 2 | 0.49 |
| 25 | 0 | 0 | 0 | 0 | 1.43 |
| 26 | 0 | 0 | 0 | 0 | 1.17 |
| 27 | 0 | 0 | 0 | 0 | 1.5 |

(a)	Fit a second-order model to the MSGase activity response.

The second-order model includes All 4 quadratic terms A2, B2, C2 and D2; the two-factor interactions AB and CD (AD is borderline and kept in the model) and the main effects C and D.  A and B were left in the model for model hierarchy.

Design Expert Output

Response	1	MTGase
	        ANOVA for Response Surface Reduced Quadratic Model
	Analysis of variance table [Partial sum of squares - Type III]
		Sum of			Mean		F	p-value
	Source	Squares		df	Square		Value	Prob > F

Model	2.49		11	0.23		10.29	< 0.0001	significant

A-X1	5.042E-004	1	5.042E-004	0.023	0.8817
	  B-X2	7.704E-003	1	7.704E-003	0.35	0.5630
	  C-X3	0.32		1	0.32		14.73	0.0016
	  D-X4	0.22		1	0.22		9.92	0.0066
	  AB	0.18		1	0.18		8.30	0.0114
	  AD	0.086		1	0.086		3.89	0.0674
	  CD	0.39		1	0.39		17.60	0.0008
	  A2	0.90		1	0.90		41.09	< 0.0001
	  B2	0.58		1	0.58		26.28	0.0001
	  C2	0.44		1	0.44		19.94	0.0005
	  D2	0.56		1	0.56		25.49	0.0001
	Residual	0.33		15	0.022
	Lack of Fit	0.27		13	0.021		0.69	0.7314	not significant
	Pure Error	0.060		2	0.030
	Cor Total	2.82		26

Std. Dev.	0.15		R-Squared		0.8830
	Mean	0.77		Adj R-Squared	0.7972
	C.V. %	19.38		Pred R-Squared	0.6452

PRESS	1.00		Adeq Precision	12.597

Coefficient		Standard	95% CI	95% CI
	Factor	Estimate		df	Error	Low	High		VIF

Intercept	1.37		1	0.086	1.18	1.55

A-X1	-4.583E-003	1	0.030	-0.069	0.060		1.00
	 B-X2	0.018		1	0.030	-0.047	0.082		1.00
	 C-X3	-0.12		1	0.030	-0.18	-0.052		1.00
	 D-X4	-0.095		1	0.030	-0.16	-0.031		1.00
	 AB	0.11		1	0.037	0.028	0.19		1.00
	 AD	-0.073		1	0.037	-0.15	5.940E-003	1.00
	 CD	-0.16		1	0.037	-0.23	-0.077		1.00
	 A2	-0.21		1	0.032	-0.27	-0.14		1.25
	 B2	-0.16		1	0.032	-0.23	-0.096		1.25
	 C2	-0.14		1	0.032	-0.21	-0.075		1.25
	 D2	-0.16		1	0.032	-0.23	-0.094		1.25

Final Equation in Terms of Actual Factors:

		 MTGase	 =

+1.36667

-4.58333E-003	  * X1
		+0.017917	  	  * X2
		-0.11625		  * X3
		-0.095417		  * X4
		+0.10688		  * X1 * X2
		-0.073125		  * X1 * X4
		-0.15563		  * X3 * X4
		-0.20594		  * X12
		-0.16469		  * X22
		-0.14344		  * X32
		-0.16219		  * X42

(b)	Analyze the residuals from this model.

Run #9 appears to be an outlier.  The residual plots look good for model adequacy.

(c)	Recommend operating conditions that maximize MTGase activity.

MTGase can be maximized with X1 = 0.03, X2 = 0.07, X3 = -0.33 and X4 = -0.14.  The MTGase response will be 1.39.

Constraints
				Lower	Upper	Lower	Upper
	Name	Goal		Limit	Limit	Weight	Weight	Importance
	A:X1	 is in range 	-2	2	1	1	3
	B:X2	 is in range 	-2	2	1	1	3
	C:X3	 is in range 	-2	2	1	1	3
	D:X4	 is in range 	-2	2	1	1	3
	MTGase	 maximize 	0.21	1.5	1	1	3

Solutions
	Number	X1	X2	X3	X4	MTGase	Desirability

1	0.03	0.07	-0.33	-0.14	1.39306		0.917	Selected

11.32.	Given the fitted response function

which is found to fit well in a localized region of (x1, x2).

(a)	Plot contours of constant response, y, in the (x1, x2) plane.

(b)	Calculate and plot the path of steepest ascent generated from this response function.  Use x1 to determine the path, with x1 = 1.

|  | Coded | Variables |  |
| --- | --- | --- | --- |
|  | x1 | x2 |  |
| Origin | 0 | 0 |  |
|  | 1 | -2.5/3.6 = -0.644 |  |
| Origin + | 1 | -0.644 |  |
| Origin +2 | 2 | -1.3888 |  |

11.33.	It is stated in the text that the development of the path of steepest ascent makes use of the assumption that the model is truly first-order in nature.  However, even if there is a modest amount of curvature or interaction in the system, the use of steepest ascent can be extremely useful in determining a future experimental region.  Suppose that in a system involving x1 and x2 the actual model is given by

Assume that x1and x2 are in coded form.

(a)	Show a plot of the path of steepest ascent (based on the actual parameters) if the interaction is ignored.

(b)	Show a plot of the path of steepest ascent for the model with interaction.  Note that this path is not linear.

(c)	Comment on the difference in the two paths.

The interaction puts the curvature into the model.

11.34S.	Suppose you have fit the following model:

(a)	The experimenter decides to choose x1 as the variable to define the step size for the steepest ascent.  Do you think that this is a good choice for defining step size?  Explain why or why not.

Usually we choose the variable we know the most about or the variable with largest absolute value coefficient.  In this case, if we know the most about x1 then we are good.  If not, then x3 may have been a better choice.

(b)	The experimenter decides to make the stop size exactly one coded unit in the x1 direction.  Find the coordinates of this point from the design center on the path of steepest ascent.

|  | Coded | Variables |  |
| --- | --- | --- | --- |
|  | x1 | x2 | x3 |
| Origin | 0 | 0 | 0 |
|  | 1 | -4/10 = -0.4 | 12/10= 1.2 |
| Origin + | 1 | -0.4 | 1.2 |

(c)	Suppose that you had used x3 as the variable to define the direction of steepest ascent.  Assuming a step size of one coded unit, find the coordinates of the first step in this direction away from the design center.

|  | Coded | Variables |  |
| --- | --- | --- | --- |
|  | x1 | x2 | x3 |
| Origin | 0 | 0 | 0 |
|  | 10/12 = 0.833 | -4/12 = -0.333 | 1 |
| Origin + | 0.833 | -0.333 | 1 |

(d)	Find the distance between the two points found in parts (b) and (c), respectively.

The two paths are 0.269 units away from each other.  Not very far.

11.35.	Any point on the path of steepest ascent is proportional to the unit vector of the first-order regression coefficients.

True  False

11.36.	Consider the first-order model

If the experimenter used one coded unit in the x1 direction as the stop size, the point x1 = 1.5, x2 = 1.0 is a point on the path of steepest ascent.

True   False

11.37.	A 23 factorial design with 4 center points has been used to fit the first-order model

The variance of the predicted response at any design point other than the center point is

11.38.	When conducting steepest ascent experiments, it may be a good idea to replicate runs at some or all of the points along the path.

True    False

11.39.	If a first-order model with interaction is used, the path of steepest ascent is a straight line.

True    False

11.40.	The coordinates of the points on the path of steepest ascent are proportional to the effect estimates from the first-order model.

True    False

11.41.	For a first-order model with three variables the variance of the predicted response at the design center (in coded units) is 2, if an unreplicated 23 design is used to collect the experimental data.

True    False

11.42.	The method of steepest ascent is a gradient procedure.

True    False

11.43.	Using a second-order model to estimate the underlying relationship is a common approach to approximating the surface.

True    False

11.44.	The location of the stationary point corresponds to a global maximum or minimum of the surface.

True    False

11.45.	The existence of a ridge system in the canonical variables can restrict the choice of optimal location.

True    False

11.46.	If the sign of all of the eigenvalues is the same, the response surface will have a global maximum.

True    False

11.47.	In designing mixture experiments, a good rule of thumb is to assume that at least a second-order mixture model is likely to be adequate.

True    False

11.48.	The simplex-lattice and simplex-centroid designs have most of the design points on the boundary of the experimental region.

True    False

11.49.	When axial check blends are added to a simplex-lattice, these blends are as far as possible (Euclidean distance) from the other design points.

True    False

11.50.	The (3,2) simplex-lattice design is a rotatable design for fitting a second-order mixture model.

True    False